# User MANUAL

MCR-ALS App v6a

Dr. András Gorzsás

Vibrational Spectroscopy Core Facility, Department of Chemistry, Umeå University, SE-90187 Umeå, SWEDEN

andras.gorzsas@umu.se

2023 April

Table of Contents

Disclaimer and Credits	5

Changelog	6

2023 April, v6	6

2022 January, v5	7

2019 September, v4cj	7

2016 September, v4c	7

2016 August, v4b	7

Terms and Conditions	9

1.	Introduction	10

1.1.	What This Manual Is	10

1.2.	What This App Is	10

1.3.	System Requirements and General Notes	11

1.4.	Data Format Requirements	11

1.4.1.	Input data	12

1.4.2.	Reference Spectra	15

1.4.3.	Initial Estimate Input	16

1.4.4.	Equality Constraint Matrices	16

1.4.5.	Pre-Processing Parameter Matrix	17

1.5.	Output Data Formats	17

1.5.1.	Pre-Processing Parameter Matrix	17

1.5.2.	Cut and Pre-Processed Spectra	19

1.5.3.	Integral and Single Point Intensity Matrices	19

1.5.4.	MCR-ALS Results	20

1.5.5.	Compiled Class Matrix	20

1.5.6.	Reference Match Results	21

1.5.7.	Segmentation Results	21

1.5.8.	Plots	21

2.	Opening the App	23

2.1 Start	23

2.2 Layout	23

3.	Pre-Processing	26

3.1 Loading the Data	26

3.2 Selecting a spectrum for display	29

3.3. Spectral Region Adjustment (Cutting)	29

3.4. Asymmetrical Least Squares Baseline Correction	31

3.5. Normalisation	33

3.5.1. Total Area	33

3.5.2. Total Min-Max	34

3.5.3. Region Area	34

3.5.4. Region Min-Max	34

3.5.5 Region Max	34

3.5.6. Point Max	34

3.5.7. Offset	34

3.6. Smoothing	35

3.7. Integration	36

3.8. Single Point Intensity Evaluation	39

3.9. Saving and Loading Pre-Processing Parameters	42

3.10. Displaying the Pre-Processed Data	42

3.11. Saving the Pre-Processed Data	43

3.12. Passing the Data for MCR-ALS	43

4.	MCR-ALS	45

4.1.	Number of Components	45

4.2.	Show PCA	47

4.3.	Initial Estimates	48

4.3.1.	Automatic Estimation and Show Purest	48

4.3.2.	Load Input	50

4.3.3.	Use Marked Spectra	51

4.3.4.	Use Reference Spectra	53

4.4.	Direction	55

4.5.	Constraints	55

4.5.1.	Non-Negativity	56

4.5.2.	Equality	56

4.5.3.	Unimodality	59

4.5.4.	Closure	61

4.6.	Number of Iterations and Convergence Criterion	62

4.7.	Perform MCR-ALS	62

4.8.	Optimised Spectra	64

4.9.	Optimised Concentrations	65

4.9.1.	Show the Component Maps	65

4.10.	Save the MCR-ALS Results	67

5.	Visualisation and Classification	69

5.1.	White Light Image	69

5.2.	Visualisation Controls	69

5.2.1.	Change Plot	69

5.2.2.	Change Colour	71

5.2.3.	Change Dimensions (Refold and Transpose)	72

5.2.4.	Oversample	74

5.2.5.	Show Purest	75

5.3.	Show Component Plots	76

5.4.	Save Comp Plots	76

5.5.	Mark Classes	77

5.5.1.	Manual Marking / Unmarking	77

5.5.2.	Mark from Selected	78

5.6.	Reset All Classes	78

5.7.	Compile Class Matrix	78

6.	Reference Matching	81

6.1.	Load Reference Spectra	81

6.2.	Match References	83

6.3.	Save Match Results	86

6.4.	Save Normalised References	86

6.5.	Use References as Initials	87

7.	Clustering and Segmentation	88

7.1.	Number of Clusters	88

7.1.1.	Manual Input	88

7.1.2.	Silhouette Clusters	88

7.2.	k-Means Clustering	89

7.3.	Save Segmentation Results	91

8.	Closing the App	92

Acknowledgements	93

# Disclaimer and Credits

The App itself is open source and free of charge but is subject to

Changelog

2023 April, v6

- Complete re-work of the code and design via MATLAB’s “Appdesigner”, due to “GUIDE” support terminating. It is now an App, a single “.mlapp” file, instead of the previous pair of “.m” and “.fig” files.

- Code is no longer written with maximum block independence and educational readability in focus. However, extensive commenting and descriptive variable / App element names are kept and the code is still often broken down to perform one task in one line to facilitate readability.

- Input in Excel format can now handle spectrum IDs (sample names, spectrum names, etc). If there were spectrum IDs in the input file, they will be retained during saving any data in Excel format (e.g. pre-processed spectra, integrals, MCR-ALS results, clustering results, etc)

- A number of “Save Plot” buttons have been removed, as they became redundant. The App now allows for plot save (and other plot controls, such as zoom and pan) directly in the interface, by hovering the mouse over the plot and selecting the desired option from the appearing set of icons. It is more powerful, as it also allows copying plots (both as images and as vector graphics).

- The buttons to individually save the MCR-ALS optimised concentration and spectral profiles were removed. These profiles can be saved together with a single button. Also, they are added to the MATLAB Workspace as COpt and SOpt variables for easy copy – paste.

- The MCR-ALS algorithm does not write the iteration details in the MATLAB command prompt any more. Details are only shown in the App.

- The “Save Cut Spectra” button was removed, as it became redundant. If the original spectra need to be saved only trimmed and not pre-processed in any other way, simply set AsLS lambda to 1, AsLS p to 1 (i.e. not performing baseline correction), Normalisation to ‘None’, and untick the “S-G Filter” checkbox (i.e. not performing smoothing either) and then save via the “Save All Pre-Processed” button. In this case, only cutting will be performed.

- Saving the processed data in Excel format will append a new worksheet, called “Processed” to the end of an already existing Excel file (if a new Excel file is created, then it is the first worksheet). If the data is only cut but not processed, the worksheet will be called “Cut”, instead of “Processed”. If any kind of processing was done, the pre-processing parameters will also be appended after the “Processed” worksheet, on a separate worksheet. This only affects saving in Excel .xlsx formats!

- Saving the pre-processing parameters now indicates if a cut was performed during processing. Pre-processing parameters saved by previous versions of the GUI can still be read by this version.

- The App can now store more than one integral / single point intensity plot in the evaluation panel. Swapping between these plots is via the “Visualised” drop-down menu. Note, however, that the integrals are identified only by their range and the normalisation performed. Any other change (baseline, cutting, smoothing) that may influence the integrals are NOT stored. Thus, performing an integral over the same range, using the same normalisation BUT a different set of other processing parameters will overwrite the integral of the same name without warning. The same is valid for the single point intensities.

- It is now possible to save all integrals / single point intensities in .mat or Excel format. In Excel, they will be added on a separate worksheet, called “Integrals” / “SinglePoint_Intensities” respectively. When saved as a .mat file, two variables are saved, one for the names (identifiers) and one for the evaluated data (integrals or single point intensities).

- Initial Estimates plot axis labels now reflect directional changes (Spectrum vs Concentration).

- Added a “Stop MCR-ALS” button, to exit the iterative loops before any of the end criteria is achieved.

- Added a “Transpose” button to quickly swap X and Y dimensions of image datasets (to correct for erroneous formatting, where X and Y coordinates are swapped, particularly for old WiRE .txt file inputs).

- It is now possible to mark 6 different classes instead of 5.

- Loading external Initial Estimates now uses Excel .xlsx files, instead of .mat files.

## 2022 January, v5

- Code reworked to account for MATLAB function changes (xlswrite, sgolayfilt, etc.)

- Support for .txt files generated by WiRE5.

## 2019 September, v4cj

- Added the possibility to save the processed spectra in Microsoft Excel .xlsx format

- Removed the GUI suggestion to include lambda and p in the filename when saving the processed Spectra

## 2016 September, v4c

- Equality constraints fixed

- Unimodality constraints fixed

- Unimodality mode selection options updated: no longer necessary to have the same mode for concentrations and spectra, they are independent. GUI layout and controls are updated accordingly (checkboxes replaced by popup lists).

- Added the possibility to save the normalised Reference Spectra

- Added the possibility to use the normalised Reference Spectra as Initial Estimates

- Closure constraint updated: If no normalisation is performed during pre-processing, spectral equal length closure constraint is applied by default during MCR-ALS. However, if any kind of normalisation has been performed, not closure is applied during MCR-ALS (scale indeterminacies are assumed to be handled by the normalisation)

- Added the possibility of no baseline correction. Use lambda=1 and p=1 if spectra need to be pre-processed without baseline correction.

- Fixed a bug that appeared during spectral cutting if the spectral data input was organised in ascending spectral variable order

## 2016 August, v4b

- Input now accepts Microsoft Excel .xlsx files

- SOpt, COpt, Wavenumbers and the Matrix4MCR variables are loaded into the MATLAB workspace, allowing direct copy & paste without saving

- Normalisation methods expanded from “None” and “Total Area Normalisation” to include “Total Min-Max”, “Region Area”, “Region Min-Max”, “Region Max”, “Point Max” and “Offset” types

- Reference spectra do not have to be identical in spectral range or resolution and do not have to match the data in those respects either. They are automatically made compatible upon loading. The compatible spectra can also be saved for later use or for use outside the GUI.

- Minor layout changes: The Save MCR Results button relocated, the oversampling and refolding controls adjusted in position

Terms and Conditions and requires MATLAB, which in turn is subject to terms and conditions by MathWorks.

MATLAB version 2022b or later is needed for all functions and the App to work, although parts of the code are possible to run on older versions.

The code and this manual are based on scripts, techniques and descriptions from the following sources and references therein (sometimes taken almost literally):

J. Felten, H. Hall, J. Jaumot, R. Tauler, A. de Juan, A. Gorzsás, Nature Protocols, 2015 (parts of the App layout)

P. H. C. Eilers, Analytical Chemistry, 2004 (Baseline correction)

A. Savitzky, M. J. E. Golay, Analytical Chemistry, 1964 (Smoothing)

J. Jaumot, R. Gargallo, A. de Juan, R. Tauler, Chemometrics and Intelligent Laboratory Systems, 2005 (MCR-ALS, including constraints and automatic initial estimate calculations)

G. Blanchet, M. Charbit, Digital Signal and Image Processing Using MATLAB, ISBN 978-1-905209-13-2, 2006 (Oversampling)

S. Piqueras, L. Duponchel, R. Tauler, A. de Juan, Analytica Chimica Acta, 2011 (Segmentation)

MATLAB documentation, http://mathworks.com/help/matlab/ (core function definitions and descriptions)

The Manual is not intended as a scientific publication or as an exhaustive resource on any functions of the App or the corresponding theories.

The demo examples have been recorded at the Vibrational Spectroscopy Core Facility, Umeå University, Umeå, Sweden and are modified versions of the datasets described in J. Felten, H. Hall, J. Jaumot, R. Tauler, A. de Juan, A. Gorzsás, Nature Protocols, 2015 (demo_image.txt and demo_indep_spectra.xlsx) and T. Gillgren, A. Gorzsás, Wood Science and Technology, 2016 (demo_series.mat).

# Changelog

## 2023 April, v6

- Complete re-work of the code and design via MATLAB’s “Appdesigner”, due to “GUIDE” support terminating. It is now an App, a single “.mlapp” file, instead of the previous pair of “.m” and “.fig” files.

- Code is no longer written with maximum block independence and educational readability in focus. However, extensive commenting and descriptive variable / App element names are kept and the code is still often broken down to perform one task in one line to facilitate readability.

- Input in Excel format can now handle spectrum IDs (sample names, spectrum names, etc). If there were spectrum IDs in the input file, they will be retained during saving any data in Excel format (e.g. pre-processed spectra, integrals, MCR-ALS results, clustering results, etc)

- A number of “Save Plot” buttons have been removed, as they became redundant. The App now allows for plot save (and other plot controls, such as zoom and pan) directly in the interface, by hovering the mouse over the plot and selecting the desired option from the appearing set of icons. It is more powerful, as it also allows copying plots (both as images and as vector graphics).

- The buttons to individually save the MCR-ALS optimised concentration and spectral profiles were removed. These profiles can be saved together with a single button. Also, they are added to the MATLAB Workspace as COpt and SOpt variables for easy copy – paste.

- The MCR-ALS algorithm does not write the iteration details in the MATLAB command prompt any more. Details are only shown in the App.

- The “Save Cut Spectra” button was removed, as it became redundant. If the original spectra need to be saved only trimmed and not pre-processed in any other way, simply set AsLS lambda to 1, AsLS p to 1 (i.e. not performing baseline correction), Normalisation to ‘None’, and untick the “S-G Filter” checkbox (i.e. not performing smoothing either) and then save via the “Save All Pre-Processed” button. In this case, only cutting will be performed.

- Saving the processed data in Excel format will append a new worksheet, called “Processed” to the end of an already existing Excel file (if a new Excel file is created, then it is the first worksheet). If the data is only cut but not processed, the worksheet will be called “Cut”, instead of “Processed”. If any kind of processing was done, the pre-processing parameters will also be appended after the “Processed” worksheet, on a separate worksheet. This only affects saving in Excel .xlsx formats!

- Saving the pre-processing parameters now indicates if a cut was performed during processing. Pre-processing parameters saved by previous versions of the GUI can still be read by this version.

- The App can now store more than one integral / single point intensity plot in the evaluation panel. Swapping between these plots is via the “Visualised” drop-down menu. Note, however, that the integrals are identified only by their range and the normalisation performed. Any other change (baseline, cutting, smoothing) that may influence the integrals are NOT stored. Thus, performing an integral over the same range, using the same normalisation BUT a different set of other processing parameters will overwrite the integral of the same name without warning. The same is valid for the single point intensities.

- It is now possible to save all integrals / single point intensities in .mat or Excel format. In Excel, they will be added on a separate worksheet, called “Integrals” / “SinglePoint_Intensities” respectively. When saved as a .mat file, two variables are saved, one for the names (identifiers) and one for the evaluated data (integrals or single point intensities).

- Initial Estimates plot axis labels now reflect directional changes (Spectrum vs Concentration).

- Added a “Stop MCR-ALS” button, to exit the iterative loops before any of the end criteria is achieved.

- Added a “Transpose” button to quickly swap X and Y dimensions of image datasets (to correct for erroneous formatting, where X and Y coordinates are swapped, particularly for old WiRE .txt file inputs).

- It is now possible to mark 6 different classes instead of 5.

- Loading external Initial Estimates now uses Excel .xlsx files, instead of .mat files.

## 2022 January, v5

- Code reworked to account for MATLAB function changes (xlswrite, sgolayfilt, etc.)

- Support for .txt files generated by WiRE5.

## 2019 September, v4cj

- Added the possibility to save the processed spectra in Microsoft Excel .xlsx format

- Removed the GUI suggestion to include lambda and p in the filename when saving the processed Spectra

## 2016 September, v4c

- Equality constraints fixed

- Unimodality constraints fixed

- Unimodality mode selection options updated: no longer necessary to have the same mode for concentrations and spectra, they are independent. GUI layout and controls are updated accordingly (checkboxes replaced by popup lists).

- Added the possibility to save the normalised Reference Spectra

- Added the possibility to use the normalised Reference Spectra as Initial Estimates

- Closure constraint updated: If no normalisation is performed during pre-processing, spectral equal length closure constraint is applied by default during MCR-ALS. However, if any kind of normalisation has been performed, not closure is applied during MCR-ALS (scale indeterminacies are assumed to be handled by the normalisation)

- Added the possibility of no baseline correction. Use lambda=1 and p=1 if spectra need to be pre-processed without baseline correction.

- Fixed a bug that appeared during spectral cutting if the spectral data input was organised in ascending spectral variable order

## 2016 August, v4b

- Input now accepts Microsoft Excel .xlsx files

- SOpt, COpt, Wavenumbers and the Matrix4MCR variables are loaded into the MATLAB workspace, allowing direct copy & paste without saving

- Normalisation methods expanded from “None” and “Total Area Normalisation” to include “Total Min-Max”, “Region Area”, “Region Min-Max”, “Region Max”, “Point Max” and “Offset” types

- Reference spectra do not have to be identical in spectral range or resolution and do not have to match the data in those respects either. They are automatically made compatible upon loading. The compatible spectra can also be saved for later use or for use outside the GUI.

- Minor layout changes: The Save MCR Results button relocated, the oversampling and refolding controls adjusted in position

# Terms and Conditions

Copyright (c) 2023 Andras Gorzsas

Installation, use, reproduction, display, modification and redistribution of this App / software / script, either in source or in binary forms, are permitted for non-commercial purposes only. Any exercise of rights under this license by you or your sub-licensees is subject to the following conditions:

1. Redistributions of this App / software / script, or parts of it, with or without modification, must reproduce the above copyright notice and the above license statement as well as this list of conditions, in the app / software / script, the user documentation and any other materials provided with the app/ software / script.

2. The user documentation, if any, included with a redistribution, must include the following notice: “This product includes codes developed at the Vibrational Spectroscopy Core Facility at Umeå University, Sweden (https://www.umu.se/en/research/infrastructure/visp/)."

Alternatively, if that is where third-party acknowledgments normally appear, this acknowledgment must be reproduced in the software itself.

3. Upon using (any part of) the code or the User Manual in presentations or publications, you are obliged to acknowledge and notify the Vibrational Spectroscopy Core Facility at Umeå University.

4. The Vibrational Spectroscopy Core Facility is under no obligation to provide anyone with any bug fixes, patches, upgrades or other modifications, enhancements or derivatives of the features, functionality or performance of this App / software / script.

THIS APP / SOFTWARE / SCRIPT IS PROVIDED BY THE VIBRATIONAL SPECTROSCOPY CORE FACILITY AT UMEÅ UNIVERSITY "AS IS" AND ANY IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY, OF SATISFACTORY QUALITY, AND FITNESS FOR A PARTICULAR PURPOSE OR USE ARE DISCLAIMED. THE VIBRATIONAL SPECTROSCOPY CORE FACILITY AT UMEÅ UNIVERSITY MAKE NO REPRESENTATION THAT THE SOFTWARE, MODIFICATIONS, ENHANCEMENTS OR DERIVATIVE WORKS THEREOF, WILL NOT INFRINGE ANY PATENT, COPYRIGHT, TRADE SECRET OR OTHER PROPRIETARY RIGHT.

THE VIBRATIONAL SPECTROSCOPY CORE FACILITY AT UMEÅ UNIVERSITY HAS NO LIABILITY TO LICENSEE OR OTHER PERSONS FOR DIRECT, INDIRECT, SPECIAL, INCIDENTAL, CONSEQUENTIAL, EXEMPLARY, OR PUNITIVE DAMAGES OF ANY CHARACTER, INCLUDING, WITHOUT LIMITATION, THE RESULTS OBTAINED BY THE SOFTWARE, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES, LOSS OF USE, DATA OR PROFITS, OR BUSINESS INTERRUPTION, HOWEVER CAUSED AND ON ANY THEORY OF CONTRACT, WARRANTY, TORT (INCLUDING NEGLIGENCE), PRODUCT LIABILITY OR OTHERWISE, ARISING IN ANY WAY OUT OF THE USE OF THIS APP / SOFTWARE / SCRIPT OR ANY OF PART OF THE CODE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

# Introduction

## What This Manual Is

The purpose of this Manual is to provide operational guidelines for the MCR-ALS App developed at the Vibrational Spectroscopy Core Facility at Umeå University and it therefore systematically lists every user control of the App and their actions.

This Manual does NOT aim to provide a comprehensive description of ANY of the techniques that are utilised in the App, be it a built-in MATLAB function (such as the Savitzky-Golay filtering) or a custom-built one (such as the MCR-ALS algorithm). For such details, consult the references in the section: Disclaimer and Credits and in the comments in the source code itself.

This Manual does not explain any of the theoretical backgrounds either, although several demo datasets are provided to allow testing how the App works in action. These datasets are used in the figures of this Manual to illustrate the steps, and thus they can be used alongside the Manual. There is one dataset to show how images are handled (“demo_image.txt”), one for a series of spectra (“demo_series.mat”) and another that contains a set of independent spectra (“demo_indep_spectra.xlsx”). One of them is in MATLAB .mat format, another is in Microsoft Excel .xlsx format, and the last one is an ASCII .txt file, to demonstrate the kinds of input possibilities and data types. These also serve as templates for formatting input data for the App. Auxiliary data is also provided with the datasets (such as reference spectra to test reference matching, white light image for the image dataset, etc.).

The code is being maintained, updated and expanded ad hoc, without fixed dates or deadlines. The Manual refers to the current version only, although a Changelog is provided to list major alterations. Known issues, actions that require special attention, or sections that contain important information are marked with the symbol / text: “ Warning/Information”.

## What This App Is

The App was originally developed as a graphical user interface (GUI) to handle hyperspectral images, in order to facilitate the analysis of vibrational microspectroscopic data of biological tissues without in-depth knowledge of spectroscopy or chemometrics. Thus, the main goal was (and still is) to provide a user-friendly interface, where different data processing parameters can be easily and interactively tested, and their effect on the outcome of the analysis determined and visualised (both for educational and scientific purposes). The main objective is to allow the “owner” of the data to process it (at least at a basic level), without having to involve a chemometrician or spectroscopist.

The App has separate controls for pre-processing the data (baseline correction, smoothing, etc), performing MCR-ALS analysis with a range of possible constraints, and to evaluate the results through visualisation, reference spectra matching and segmentation. Each of these are optional, and data can be saved and exported at any step (e.g. in Microsoft Excel format), for sharing or further processing by other means (e.g. classified spectra can be used in OPLS-DA analysis by e.g. Umetrics’ SIMCA-P software).

Special care was taken to write the code in a manner that is easy to follow: most operations are detailed step-by-step in the code, and most functions are heavily commented. Care has been taken to perform only one action per line. Thus, the code is optimised for easy understanding and flexibility, and NOT for speed, although the aim was to opt for the fastest solutions whenever possible (by e.g. pre-defining variable sizes for quick memory allocation, etc).

While the App was originally developed for vibrational spectroscopic images, it can handle all spectroscopic datasets, not only images and not only vibrational spectra (although the code, comments and this Manual will use “Wavenumbers” for the spectral variables), as long as they meet the Data Format Requirements.

## System Requirements and General Notes

For editing, MATLAB is required. Version 2022b or later is required for all functions to work, although core features are possible to run on older versions too. The App itself runs on very modest hardware architectures (if MATLAB can run, so can the App), but it can be sluggish, especially for large datasets and Microsoft Excel import / export.

Warning/Information: The App contains lots of elements and although it can be re-scaled, it is ideal on screens with resolutions at 1920 x 1200 or higher. On smaller displays, controls could be partially covered or illegible in full screen mode. In those case, use the App in a smaller window, and scroll through the interface instead.

Warning/Information: MATLAB’s Appdesigner is rather wonky when it comes to resizing, especially when going to full screen view. If the window needs to be made bigger / smaller, it is recommended to drag the window edges instead of clicking on the maximise icon in the top of the window to scale it to full screen.

Warning/Information: Resizing / repositioning the App main window may not be possible when running through Microsoft Remote Desktop on small screens.

Warning/Information: If pop-up dialog windows are not closed, plots in the main App window may render incorrectly or not at all. Wait for all pop-up dialog windows to close, or close them manually. If the plots in the main window of the App have been incorrectly rendered, it may be necessary to re-issue the last command (e.g. click the last button) or change any control that influences the incorrectly rendered plot (e.g. change the colour of the Visualised plots, etc.) In the worst case, the entire App needs to be restarted.

## Data Format Requirements

Data format requirements MUST be met in order for the App to work. Changing any of these requirements requires extensive modification of the source code, due to convoluted interdependencies.

Warning/Information: There is no fail-safe mechanism in the App for detecting and correcting wrong input data formats. The App will simply crash, with error messages shown in the MATLAB Command Prompt.

Warning/Information: Mac computers and MATLAB are case sensitive. Try to use only lower case extensions for all file types (i.e. “.mat”, not “.MAT”).

Warning/Information: Use only standard Latin alphanumeric characters and underscores in filenames to avoid potential problems. Avoid spaces or any special characters, including accented letters, parentheses, etc. in filenames.

Warning/Information: Make sure regional settings do not interfere with each other (decimal dots are preferred over decimal commas).

### Input data

#### .mat file input

The input .mat file must be of the following structure (default with Bruker's OPUS 7 export into Matlab 4 format):

4000 0.0473 0.0498 0.0953 ... 0.0622

3998 0.0423 0.0123 0.1022 ... 0.0817

3996 0.0494 0.0331 0.1169 ... 0.0724

.....     ......      ......       ......      ... ......

400   0.0512 0.0678 0.0744 ... 0.0688

The first column is the wavenumbers, and thereafter each column is one spectrum (see the file “demo_series.mat”). For spectral series, spectra should be in order, there is no possibility to re-order them in the App, although refolding for visualisation is possible (see Change Dimensions (Refold and Transpose)). For image datasets, each spectrum represents one pixel / voxel, with the 1st row 1st column spectrum being the first in the list (bottom left corner in Cartesian axes), followed by 1st row 2nd column spectrum, and so on. As with spectral series, re-ordering the spectra is not possible. Re-folding is allowed, but it is not recommended for images, unless X and Y coordinates are swapped, in which case Transposing is sufficient (see Change Dimensions (Refold and Transpose)).

Warning/Information: Since the .mat files formatted this way do not contain X and Y coordinates, the .mat filename MUST EITHER HAVE the following format: AAAxBBB_filename.mat to describe the X and Y dimensions, with AAA denoting the number of pixels in the X dimension and BBB denoting the number of pixels in the Y dimension (e.g. 064x064 –note the leading zeros–, or 128x128), OR the user MUST ENTER THE CORRECT X and Y dimensions manually in a pop-up dialog box. The pop-up dialog box by default suggests X = total number of spectra in the dataset, and Y = 1.

Warning/Information: This file format loads the fastest and is recommended for large data files.

#### .xlsx file input

The input .xlsx file must be one of the following 2 structures:

- Without names

4000 	3998	3996	... 400

0.0473	0.0498	0.0953	... 0.0622

  - 0.0123	0.1022	... 0.0817

0.0494	0.0331	0.1169	... 0.0724

......	......	......	... ......

0.0512	0.0678	0.0744	... 0.0688

- With names

4000 	3998	3996	... 400

Spect.name 1	0.0473	0.0498	0.0953	... 0.0622

Spect.name 2	0.0423	0.0123	0.1022	... 0.0817

Spect.name 3	0.0494	0.0331	0.1169	... 0.0724

……		......	......	......	... ......

Spect.name N	0.0512	0.0678	0.0744	... 0.0688

The first row is the wavenumbers, thereafter each row is one spectrum (see the file “demo_indep_spectra_nonames.xlsx”). If spectral names are included, they MUST be in the first column, and then wavenumbers start from the second column first row. In this case, the first column first row cell (A1) is left empty. For spectral series, spectra should be in order, there is no possibility to re-order them in the App, although refolding for visualisation is possible (see Change Dimensions (Refold and Transpose)). For image datasets, each spectrum represents one pixel, 1st row 1st column spectrum being the first in the list (bottom left corner in Cartesian axes), followed by 1st row 2nd column spectrum, and so on. As with spectral series, it is not possible to re-order the spectra in the App. Re-folding is allowed, but it is not recommended for images, unless X and Y coordinates are swapped, in which case Transposing is sufficient (see Change Dimensions (Refold and Transpose)).

Warning/Information: Spectra MUST be in the first worksheet of the Excel file. The App will only read in THE FIRST WORKSHEET of the Excel file, meaning that other information can safely be stored in additional worksheets without causing problems at data load.

Warning/Information: USE DECIMAL DOTS ONLY. Other regional settings, such as decimal commas, will result in errors at load.

Warning/Information: Since the .xlsx files formatted this way do not contain X and Y coordinates, the .xlsx filename MUST EITHER HAVE the following format: AAAxBBB_filename.xlsx to describe the X and Y dimensions, with AAA denoting the number of pixels in the X dimension and BBB denoting the number of pixels in the Y dimension (e.g. 064x064 –note the leading zeros–, or 128x128), OR the user MUST ENTER THE CORRECT X and Y dimensions manually in a pop-up dialog box. The pop-up dialog box by default suggests X = total number of spectra in the dataset, Y = 1.

Warning/Information: This data format is the easiest to append with additional information (in extra worksheets) and is thus the best choice to process independent spectra. Generally, it is also the easiest to append with additional information and share. However, it is by far the slowest to load and save, and is therefore NOT RECOMMENDED for large datasets.

#### .txt file input (images only, no series)

The input .txt file must be of one of the following structures (defaults with Renishaw's WiRE3 and WiRE5 exports into .txt format):

- (WiRE3 export)				B) (WiRE5 Export)

1	1	1800	4528			#X	#Y	#Wave	#Intensity

1	1	1799	4321			1	1	1800	4528

1	1	1798	4413			1	1	1799	4321

.	.	....	....			.	.	…	…

1	2	1800	4619			2	1	…	…

1	2	1799	4721			2	1	…	…

The first two columns are the X and Y coordinates (in reverse order in WiRE3 and WiRE5!) of the pixel, the third column contains the wavenumbers and the fourth the intensities (see the file “demo_image.txt”). In WiRE5 exports, the first row contains headings with hashtags as well.

Warning/Information: This is an extremely inefficient format with lots of unnecessary redundancy, and therefore it loads VERY slowly. Thus, large datasets should preferentially be in .mat format. The only advantage of this .txt format (apart from being the native export of Renishaw’s WiRE software), is that X and Y dimensions do not have to be supplied (either manually or in the filename, as for .mat or .xlsx files).

Warning/Information: If the hashtag headings are removed from a WiRE5 .txt file, it will still load, but the App will consider it a WiRE3 file and the resulting image will have its X and Y coordinates swapped. Use the “Transpose” button (see Change Dimensions (Refold and Transpose)) to quickly swap X – Y coordinates, to get the correct image in that case.

#### White light image input

For image datasets, the corresponding visible (white light) map can be automatically loaded, if the file is in the same folder as the data file and has the exact same file name with .jpg extension. Any other image file supported by MATLAB can also be used, but they have to be manually loaded, using the “Load White Light Image” button in the Evaluation Panel of the App (see White Light Image).

### Reference Spectra

Warning/Information: Reference spectra will NOT be pre-processed by the pre-processing parameters. However, reference spectra will be made compatible with the dataset automatically upon loading and will be total area normalised and scaled for display. Compatibility means that reference spectra will be either trimmed to the length of the dataset (i.e. excluding all wavenumbers outside the spectral region of the dataset) or/and expanded to match the length of the dataset (by adding zeros, i.e. baseline). In addition, spectral resolution will be adjusted to match that of the dataset, using the built-in “spline” interpolation algorithm of MATLAB.

#### .mat or .txt file input for Reference Spectra

Each file should contain a single spectrum, first column being wavenumbers, second column intensities (see the reference files supplied as .txt files in the “References” folder). Multiple files are allowed to be selected for multiple reference spectra to be loaded, but they all have to be in the same folder. The file names (without path) will be used for labelling the reference spectra.

#### Microsoft Excel .xlsx file input

Warning/Information: This is the preferred option to load reference files.

Only one file is allowed, which should contain all reference spectra, in the first worksheet.

Each spectrum should take two rows: the first row is the wavenumbers and the second row is spectral intensities. The spectrum name should be given in the first column in the same row as their corresponding intensities (not their wavenumbers!). These names will be used for labelling the reference spectra.

Example (“< >” denoting an empty cell in the Excel worksheet, each “<xxx>” denoting cells with content, and “…” denoting repetition):

< >		<4000>	<3999>	<3998>	…	<400>

<Reference1>	<0.298>	<0.296> <0.295> …	<0.199>

< >		<3900>	<3880>	<3860>	…	<380>

<Reference2>	<0.143>	<0.123>	<0.133>	…	<0.102>

This example contains 2 reference spectra, with names “Reference1” and “Reference2”, with different spectral ranges and spectral resolutions.

See the included “demo_references.xlsx” file for an example.

### Initial Estimate Input

All data should be in the first worksheet of a single Microsoft Excel .xlsx file. Non-continuous ranges will be ignored (i.e. do not leave gaps / empty cells).

For spectrum input, the file must have as many rows as the number of components +1 (for the wavenumbers in the first row) and as many columns as there are wavenumbers in the dataset + 1 (for the component names in the first column). In other words, each input spectrum should be one row, with the first row being the wavenumbers, and first column should be the name / identifier of the spectrum. See the “InitEstimates_spectra.xlsx” file for an example.

For concentration input, the file must have as many rows as the number of spectra in the dataset + 1 (for the component names in the first row), and as many columns as there are components in the MCR-ALS model. Thus, each component concentration profile should be one column. The first row is the name of the component, thereafter each row represents one spectrum (i.e. the concentrations of each component in that spectrum). Component names must contain at least one letter, cannot be numerical only (i.e. “Comp1” is correct, “1” is not). See the included “InitEstimates_conc.xlsx” file for an example.

Warning/Information: Spectra should have the same length as the spectra in the dataset (same spectral range AND spectral resolution, i.e. the same set of wavenumbers), otherwise it will not be loaded. If the spectra to be used for initial estimate inputs have different spectral regions / spectral resolution, load them as reference spectra first (see Microsoft Excel .xlsx file input for Reference spectra input format requirements). This will make them compatible with the dataset automatically and they can be passed on as Initial Estimates directly using the dedicated button (see Use References as Initials), or saved (using the Save Normalised References button) and re-loaded at a later time.

Warning/Information: 3.12. Passing the Data for MCR-ALS (whether processed or not) will automatically clear all initial estimate inputs.

Warning/Information: Changing the Direction will automatically clear all initial estimate inputs.

### Equality Constraint Matrices

All data should be in a single .mat file.

If spectra equality is used, the .mat matrix must contain as many rows as the number of components to be resolved, and as many columns as there are wavenumbers in the dataset. For all the components that are to be constrained, the row should contain numbers (i.e. spectral intensities at each wavenumbers), for unconstrained components (or spectral regions), the row (and columns) should contain either NaN or Inf values.

If concentration equality is used, the .mat matrix must contain as many columns as the number of components to be resolved, and as many rows as there are spectra in the dataset. For the components to be constrained, the column should contain numbers describing the concentration of that component in every spectrum in the dataset where the concentration is to be constrained. The column should contain NaN or Inf values for the spectra where the concentration is left unconstrained. For the components that are left unconstrained, their column should contain either NaN or Inf values only.

See the included “csel.mat” and “ssel.mat” auxiliary data files for examples.

Warning/Information: Changing the Number of Components will automatically clear all loaded equality constraint matrices.

### Pre-Processing Parameter Matrix

Only use a .mat file that was saved by the App or its previous version GUI or has the exact same list of parameters in the exact same format (see

Pre-Processing Parameter Matrix output format and the “demo_image_PreProcessParam.mat” auxiliary data file).

Warning/Information: The App can open pre-processing parameter matrix .mat files that were saved with any previous versions of the App or the GUI.

## Output Data Formats

### Pre-Processing Parameter Matrix

All pre-processing parameters are saved in a single .mat file, containing a 4x13 cell matrix called “Param2Save”. Parameters are listed in this variable in pairs, with the name of the parameter on top, and the corresponding value directly below. The parameters that are saved are as follows (in the order they appear in the .mat file).

“Minimum Spectral Range”: the low wavenumber end of the spectral region

“Maximum Spectral Range”: the high wavenumber end of the spectral region.

“AsLS baseline lambda”: the lambda value of asymmetric least squares baseline correction.

“AsLS baseline p”: the p value of asymmetric least squares baseline correction.

“Normalisation”: the type of normalisation performed. The following notations can appear in the cell below:

None (referring to no normalisation)

Total Area (meaning total area normalisation)

Total MinMax (meaning minimum – maximum normalisation in the entire spectral range)

Region Area (meaning area normalisation in a defined region of the spectrum only. The region is defined by the Integral Low Edge and Integral High Edge values)

Region MinMax (meaning minimum - maximum normalisation in a defined region of the spectrum only. The region is defined by the Integral Low Edge and Integral High Edge values)

Region Max (meaning maximum intensity normalisation in a defined region of the spectrum only. The region is defined by the Integral Low Edge and Integral High Edge values)

Point Max (meaning maximum intensity normalisation at a defined point in the spectrum. The point is defined by the Single Point Intensity at value)

Offset (meaning offset correction at a defined point in the spectrum. The point being defined by the Single Point Intensity at value)

“Savitzky-Golay (S-G) Smoothing”, 1 for Savitzky-Golay filtering (smoothing) to be performed, 0 for no Savitzky-Golay filtering (smoothing) to be performed

“S-G Polynomial Order”, the polynomial order for Savitzky-Golay filtering (smoothing)

“S-G Frame Nr”, the frame number for Savitzky-Golay filtering (smoothing)

“Integral Low Edge”, the low edge of the integral range

“Integral High Edge”, the high edge of the integral range

“Show Integral”, 1 for showing the integral area in the Selected Spectrum plot, 0 for not showing it

“Single Point Intensity at”, the wavenumber at which the single point intensity should be evaluated

“Show Single Point Intensity”, 1 for showing the single point intensity marker line in the Selected Spectrum plot, 0 for not showing it

“Original Minimum Spectral Range”, the low wavenumber end of the original datafile, before any potential cutting

“Original Maximum Spectral Range”, the high wavenumber end of the original datafile, before any potential cutting

Warning/Information: The Original Minimum Spectral Range and Original Maximum Spectral Range values were not stored in previous versions of the APP or the GUI.

Warning/Information: The App can open Pre-Processing Parameter Matrix files saved by previous versions of the GUI, but the GUI cannot open Pre-Processing Parameter Matrix files saved by the App (no backward compatibility).

### Cut and Pre-Processed Spectra

Irrespective of the original input format, cut spectra can be saved in two formats:

- As a .mat file, where the first column is the wavenumbers, and thereafter each column is one spectrum. This .mat file can be read in directly by the App. Example:

4000	0.0473	0.0498	0.0953	...	0.0622

3998	0.0423	0.0123	0.1022	...	0.0817

....	......	......	......	...	......

400	0.0512 	0.0678	0.0744	...	0.0688

- As a Microsoft Excel .xlsx file in the following format (“< >” denoting an empty cell in the Excel worksheet, each “<???>” denoting cells with optional content, <xxx> denoting cells with numerical content and “…” denoting repetition):

< >	<4000>	<3999>	<3998>	…	<400>

<???>	<0.298> <0.296>	<0.298>	…	<0.1999>

....	......	......	......	...	......

<???>	<0.143>	<0.123>	<0.133>	…	<0.1023>

The first row is the wavenumbers, and thereafter each row is one spectrum. The first column may contain spectrum identifiers (names), if these were present at loading, otherwise the entire first column will be omitted, and the Excel file will only contain numerical values. This .xlsx file can be read in directly by the App.

Warning/Information: Saving large datasets to Excel format can be very slow. For large datasets, .mat format is recommended.

Warning/Information: The .mat or .xlsx files do NOT contain XY dimension information (for image data sets).The XY dimensions can be added to the filenames during the save dialog box. The App automatically suggests a dimension based on the XY dimensions specified at Load.

### Integral and Single Point Intensity Matrices

Only one integral and one single point intensity matrix can be saved at a time. The result is a .mat file with a single MATLAB matrix variable (double), called “Int” (for integrals) or “SPointInt” for single point intensities, containing a single column of integral / single point intensity values and as many rows as there are spectra in the dataset.

Warning/Information: The .mat file does NOT contain XY dimension information (for image data sets) or filenames (for spectra series) or the integral range / single point intensity position. The XY dimension and the integral range / single point intensity position can be added to the filename upon saving (default suggestion by the App) but spectrum names must be saved separately for spectra series.

### MCR-ALS Results

The end results of MCR-ALS optimisations can be saved in a single .mat file, which contains the following MATLAB variables:

“COpt”, containing the resolved concentrations profiles, consisting of as many rows as there are spectra in the dataset, and as many columns as the number of resolved components. Each column contains the concentration value of a resolved component in each spectrum.

“Ending”, containing a number to describe whether convergence was reached (“1”), divergence occurred (“2”) or the maximum number of iterations were reached (“3”).

“Iteration”, containing a number, denoting the number of iterations after which MCR-ALS was finished.

“R2Opt”, containing a number, denoting the percentage of variance explained when MCR-ALS was finished (1.00 being 100 percent)

“SDOpt”, containing two numbers denoting the lack of fit, the first in % PCA, the second in % experimental data.

“Sigma”, containing a number, denoting the change in sigma value

“SOpt”, containing the resolved spectral profiles, consisting of as many rows as there were components, and as many columns as there are wavenumbers. As opposed to saving the Error! Reference source not found., this variable does NOT contain the wavenumber list!

Alternatively, the results can be saved in Excel, with all the above parameters organised and labelled in a specific worksheet. Additionally, the Excel file retains the spectrum IDs if those were included in the input file (spectrum name, etc).

Warning/Information: The saved files do NOT contain XY dimension information (for image data sets) and only contain filenames / spectrum IDs when saved as an Excel file. The XY dimension can be added to the filename upon saving (default suggestion by the App).

Warning/Information: The resulting files do NOT contain information about the starting conditions of the MCR-ALS procedure (direction, noise level, initial estimates, and convergence criterion), only the end results.

Warning/Information: The resulting files do NOT contain information about the constraints used for the MCR-ALS procedure (equality or unimodality).

### Compiled Class Matrix

All marked spectra are compiled into the first worksheet of a single Microsoft Excel .xlsx file. The first row contains the wavenumbers, the first column the names (the original dataset name, plus the spectrum number), the second column their assigned class (class IDs), and each row is one spectrum.

This file can be directly imported into e.g. SIMCA-P, using the first column as Primary Observation ID and the first row as Primary Variable ID. The second column can be used as Y variable or class ID or secondary observation ID, or left unused.

Warning/Information: This file can NOT be directly used / loaded as Reference Spectra, since it does not match the input format criteria for Microsoft Excel .xlsx file input without adjustments.

### Reference Match Results

The results of reference matching are saved in the first worksheet of a single Microsoft Excel .xlsx file as a table with descriptive headings. Each row is one resolved component, each column is one reference spectrum, and each value of the table describes how well the component and reference matches, based on Euclidean distances.

Warning/Information: The file does NOT contain the resolved components spectral profiles OR the reference spectra, and it does not contain ANY plots to visual validation of the match.

### Segmentation Results

The results of k-means clustering can be saved in a single .mat file, which contains the following MATLAB variables:

“Centr”, containing the centroid profiles, i.e. the contribution of each resolved component to cluster, and thus consisting of as many rows as there are clusters and as many columns as the number of MCR-ALS resolved components.

“IDX”, containing the cluster identifiers, i.e. a single number to denote which cluster each spectrum in the dataset belongs to. Thus, it consists of a single column with as many rows as there are spectra in the dataset.

Alternatively, the results can be saved in an Excel file, which contains descriptive headings AND retains all spectrum IDs that were present at loading.

Warning/Information: The files do NOT contain XY dimension information (for image data sets). The XY dimension can be added to the filename upon saving (default suggestion by the App).

### Plots

Plots of the main interface of the App and from any figure window opened by the App can be saved by hovering over the plot with the mouse and selecting the save options from the appearing context sensitive set. In addition, figures can be copied, both as image or as vector graphic. There are no separate save buttons for any of the individual plots.

Figure 1: The context sensitive save / copy plot icons, as appearing when hovering over the Original Spectra Plot, with the demo_image.txt file loaded.

The Save Comp Plots button saves ALL the component concentration maps AND the component spectral profiles individually, as separate files. They can be saved as image (.jpg, .png and .tif formats) or as vector graphics (.eps, .pdf). This button saves all plots in the same format and in the same folder.

Warning/Information: The Save Comp Plots button saves all plots in the same format and in the same folder. It is not possible to specify different formats / folders for individual plots in this case.

Warning/Information: Mac and Windows compatibility and resolution problems of the saved plots may arise (especially for .tif files). Test different formats to find the optimal solution to the current computer architecture.

# Opening the App

## 2.1 Start

Start MATLAB and navigate to the MCRALS_v6a_App.mlapp file in the browser pane. Right-click on the file and select “Run” from the opening context sensitive menu.

If the folder in which the MCRALS_v6a_App.mlapp file resides is not in the MATLAB path, a dialog box opens. Choose the “Change to Folder” option.

Warning/Information: MATLAB shows a set of warning messages in the Command prompt at start up, which are automatically cleared by the App. These only refer to built-in MATLAB restraints of slider bar size scaling and have nothing to do with the functions of the App.

## 2.2 Layout

Figure 2: The empty App at start

The App is designed in the way that controls lacking input value are disabled by default. Thus, at start only the “Load Data” button is active (Figure 2). As the interface populates, more options become available.

Warning/Information: The App contains lots of elements and although it can be re-scaled, it is ideal on screens with resolutions at 1920 x 1200 or higher. On smaller displays, controls could be partially covered or illegible in full screen mode. In those case, use the App in a smaller window, and scroll through the interface instead.

Warning/Information: MATLAB’s Appdesigner is rather wonky when it comes to resizing, especially when going to full screen view. If the window needs to be made bigger / smaller, it is recommended to drag the window edges instead of clicking on the maximise icon in the top of the window to scale it to full screen.

Warning/Information: When using Microsoft Remote Desktop to connect to a different PC, the screen resolution may not be transferred correctly and the App window may fall completely or partly outside of the view. This may make it impossible to use the App through such connections.

The App interface can be divided into 3 major panels (Figure 3), and while each are optional, they may depend on each other for input.

1

2

3

Figure 3: The empty App at start, with the Panels marked

Panel 1: Pre-Processing, also including controls for loading the data, saving processed-data and passing on the data for MCR-ALS (Panel 2) and Evaluation (Panel 3).

Panel 2: MCR-ALS, including controls for determining the number of components, determining the initial estimates (with a possibility to use marked spectra from Panel 3), setting MCR-ALS constraints and parameters and performing MCR-ALS analysis. The results are automatically passed on for visualisation to Panel 3.

Panel 3: Evaluation, including visualisation controls, marking and assigning classes (with a possibility to select the displayed spectrum from Panel 1 directly), exporting the marked classes or using them as initial estimates for MCR-ALS (in Panel 2), refolding and oversampling and changing the colour scheme of any of the visualisation plots. Furthermore, it includes controls for reference spectra matching, based on Euclidean distances (which requires a completed MCR-ALS analysis from Panel 2) AND loaded reference spectra for input. Controls for segmentation, based on k-means clustering are also included, and it also requires a completed MCR-ALS analysis from Panel 2, for input. Finally, Credits are shown, with a possibility to copy the text (for e.g. Google search).

Each Panel is discussed in details in the chapters below.

Warning/Information: The App uses colour coding for the buttons: black text means the button is a regular control; blue text means the button saves the data in the way that it can be later read in by the App directly, without modification; red text means the button can be critical in passing information to another part of the App. Texts to radio buttons to Mark Classes are shown in the colour of the class they are marking.

# Pre-Processing

## 3.1 Loading the Data

Click on the “Load Data” button. If there was no data loaded before, a standard dialog box opens, showing the folder where the App resides, and all .mat files in that folder by default. Change folder and file type as needed.

Warning/Information: In certain cases, the Finder in Mac OSX does not update upon changing the file type in the dialog window, and the newly selected file types remain greyed out. This only affects the look of the dialog window, not its function. Thus, the file types can still be opened even if they are greyed out. Alternatively, selecting “all files” in the file type option or changing the folder in the dialog window will force Finder to update the display.

If there was a dataset loaded already, loading a new dataset will completely overwrite all the previous data in the App. It will NOT affect the original and saved files, only the data that is not saved. A Warning/Information Dialog window opens to inform about this and allowing to cancel before old data is wiped out. Pre-processing and MCR-ALS parameters, however, are kept in the App, thus the new dataset can quickly be processed in the exact same way as the previous dataset was, without having to reset (or reload) all parameters.

Warning/Information: To make sure that no carryover variable affects a completely new dataset, it may be necessary to close the entire App, type “clear all” in the MATLAB Command Window and restart the App (see Opening the ).

Select the file to be opened and either double-click it, or click on the “Open” button in the dialog window.

Warning/Information: .mat and .xlsx files must either contain the X and Y dimensions in the filename (see .mat file input) or the correct dimensions must be supplied in the opening dialog box. For image files, it is especially important to have the correct X and Y dimensions otherwise the data will be folded wrongly. For independent spectra or a series of spectra it is only important that X*Y equals the total number of spectra in the dataset. The App suggests X and Y dimensions by default, with X = total number of spectra in the dataset, and Y = 1 (Figure 4).

Figure 4: X and Y dimension dialog window, to manually supply the requested dimension for .mat and .xlsx files that do not contain this information in the filename. By default, the Y dimension is set to 1, and the X dimension to the total number of spectra in the dataset (in this case 15). The example uses the “demo_indep_spectra.xlsx” file.

Warning/Information: There is no mechanism to correct for erroneous X and Y input. With wrong dimension supplied, or the “Cancel” button hit in the X and Y dimension dialog window (Figure 3), the App will simply crash with error messages shown in the MATLAB command prompt. In this case, it is best to close and restart the App (see 2.1 Start).

When the data is loaded, the filename is shown in the “File in Use:” title, and spectra are shown in the Original Spectra Plot of the Pre-Processing panel of the App (first plot on the top left). (Figures 5 and 6).

Warning/Information: If the dataset contains more than 100 spectra, only 100 randomly selected ones are shown in the Original Spectra window, to speed up the App (Figure 5).

Warning/Information: Large files may take a long time to load, especially on slower computers and in case of .txt or especially .xlsx files.

The rest of the Pre-Processing panel of the App gets populated, and controls for spectrum selection (see 3.2 Selecting a spectrum for display), spectral region trimming (see 3.3. Spectral Region Adjustment (Cutting)), baseline correction (see 3.4. Asymmetrical Least Squares Baseline Correction), Savitzky-Golay filtering (3.6. Smoothing), normalisation (see 3.5. Normalisation), intensity determination (either as band area, see 3.7. Integration, or as a single value, see 3.8. Single Point Intensity Evaluation) become active.

The Evaluation panel of the App shows the total intensity map of the dataset (i.e. the total area under all peaks in the spectrum, without pre-processing). The “Visualised” drop-down menu is activated and shows “Orig Tot Int” to reflect this fact. The “Colours” drop-down menu is activated, allowing to change the colour map of the visualised plots. The default set is “Copper” to avoid confusions with MCR-ALS component colours (see Change Colour for the list of available colour maps) (Figures 5 and 6).

If a hyperspectral image dataset was loaded, a corresponding visible (white light) image is loaded by default (Figure 5), provided that it has the same filename as the data file, with .jpg extension, and that it resides in the same folder as the loaded data. If that is not the case, a visible image can be loaded manually by the “Load White Light Image” button in the Evaluation panel of the App (see White light image input).

The controls for image refolding also become active, showing the current X and Y coordinates in the “New X” and “New Y” text boxes, with the “Transpose” and “Refold” buttons activated underneath (see Change Dimensions (Refold and Transpose)).

The Oversampling option also becomes active, allowing for mathematical refining of the image (virtual resolution increase, see Oversample).

The “Mark Classes” radio buttons are also activated, together with the “Mark from Selected” button, allowing for manual labelling of selected spectra (assigning them to different classes, see Mark Classes).

Figure 5: An image dataset is loaded from a .txt file. This dataset contains more 125 spectra, of which only 100 randomly selected spectra are shown in the Original Spectra Plot (first plot in the top left of the App), in MATLAB’s “parula” colour map. The example uses the “demo_image.txt” file (with “demo_image.jpg” automatically loaded if it resides in the same folder as the “demo_image.txt” file).

Figure 6: A spectrum data set is loaded. This data set contains 15 independent spectra (not in a series). Since the total number is less than 100, all of them are loaded at once, but no corresponding white light image is loaded (upper right). The example uses the “demo_indep_spectra.xlsx” file.

## 3.2 Selecting a spectrum for display

Once the data is loaded (see 3.1 Loading the Data) the “Selected Spectrum” plot is populated with either the first spectrum in the set (if there are less than a 100 spectra in total), or a randomly selected spectrum (if the dataset contains more than a 100 spectra). A drop-down menu and a text box show the number of the spectrum being displayed in the plot. The selected spectrum is shown in red, the baseline (see 3.4. Asymmetrical Least Squares Baseline Correction) in dark grey and the resulting baseline corrected spectrum in black (Figure 7).

Warning/Information: If the dataset contains more than 100 spectra, only the 100 randomly selected ones that are shown in the Original Spectra Plot are listed in the Selected Spectrum drop-down menu. In this case, the same spectrum can feature multiple times in the drop-down list.

ANY spectrum can be manually selected by typing the spectrum number in the “Selected Spectrum” textbox (even if it does not feature in the drop-down menu originally, which can happen in the case there were more than 100 spectra in the dataset). This spectrum is then added to the drop-down list and can be assigned to any class by the “Get From Selected” button in the Evaluation Panel of the App (see Mark from Selected), and from there it can be used as an initial estimate by the “Use Marked” button in the MCR-ALS Panel of the App (see Use Marked Spectra).

Figure 7: The Selected Spectrum plot. Red is the original spectrum (nr. 102 in the series, as shown in the drop-down list and the text box), grey is the baseline (clearly underfitted in this case, see Figure 9), and black is the resulting, baseline-corrected spectrum. The example uses the “demo_image.txt” file.

## 3.3. Spectral Region Adjustment (Cutting)

It can be beneficial to trim the spectral region, to eliminate noise or artefacts, or to facilitate baseline correction (see 3.4. Asymmetrical Least Squares Baseline Correction).

This can be achieved by cutting the lower, higher or both ends of the spectral region, using the controls provided in the “Spectral Region” box of the Pre-Processing Panel of the App.

Warning/Information: The sliders may be hard to set exactly. For precise values, use the textboxes.

Warning/Information: Only the low and high ends can be trimmed, it is not possible to cut an arbitrary region from within the spectra.

Warning/Information: Cutting can only be done in the X variable space (i.e. in the spectral dimension), NOT in the observation space (no image pixel or individual spectrum can be excluded from the dataset in the App)

The region to be cut is marked in semi-transparent yellow colour and can be adjusted by the sliders or directly by typing the desired value in the corresponding text box (Figure 8) and hitting the “Enter” (PC) or “Return” (Mac) key. The cutting is only performed after clicking the “Cut” button. The resulting spectrum will only contain data that falls within the range specified, and the low and high edge edit fields and the corresponding sliders will display the new minimum and maximum values (Figure 8). The “Selected Spectrum” plot will automatically update, along with the “Marked Spectra” plot in the Evaluation Panel of the App, if there are any marked spectra plotted there already (see Mark Classes).

Warning/Information: When a new dataset is loaded, which contains a broader spectral range than the previously cut one, the cutting limits are kept in the App. Thus, the new dataset can be quickly trimmed to the same zone as the previous dataset, by clicking the “Cut” button, without having to set the low and high edges again. However, if the new dataset contains a spectral region with any or both ends WITHIN the cut limits (i.e. a narrow range than the previous cut), the cut limits will be overwritten with the new minimum/maximum/both spectral regions limits as the spectral region can only be cut, and NOT expanded (spectra cannot be padded to expand the spectral region on either ends).

The cutting can be undone by clicking the “Reset” button, which restores the original spectral region.

Warning/Information: There is no stepwise undo. After multiple cuts, the dataset is restored to its original spectral region, not to the result of the previous cut.

Warning/Information: Changing the spectral dimensions makes the data incompatible with many parts of the App. Incompatible controls are disabled and data cleared, without warning, including Reference spectra (if there were any), set constraints (including their loaded matrices), initial estimates, etc. MCR-ALS cannot be started until the new data with the different spectral region is passed again (3.12. Passing the Data for MCR-ALS). If, however, MCR-ALS has already been performed before, those results are kept until the data is passed again, to allow for saving. (This includes plots, the “Purest” listbox, the number of components, etc.)

Figure 8: Cutting the spectral region: before (left) and after (right). The region was set to 580 – 1780 cm-1 (left). The new dataset contains only points within this region, the first recorded data being 580.391 cm-1, and the last 1779.34 cm-1 (right) in the present example, which uses the “demo_image.txt” file. Note how the sliders updated and the “Reset” button became active.

## 3.4. Asymmetrical Least Squares Baseline Correction

Baseline correction is performed via asymmetrical least squares fitting, a method developed by Eilers (Analytical Chemistry, 2004). It uses only two parameters to perform a fit to each spectrum in the dataset: lambda and p. This baseline correction method does not operate with a pre-defined baseline shape, but fits a baseline to each spectrum individually. The lambda value determines how close the baseline needs to follow (fit) the original data. At minimum value, it follows very closely (Figure 9 left), generally resulting in severe overfitting. At maximum value, it produces a linear baseline, connecting the first and last data point in the spectrum, which can result in underfitted baselines (Figure 9 middle). The p value determines the weight of negative contributions to the sum of fit (hence the name asymmetrical). In practical terms, minimum p value does not allow negative data during baseline fit, i.e. the baseline is not allowed to “cut through” the spectrum. In other words, all points in the spectrum must be positive, as no point can be below the baseline (Figure 9 middle). This should be the default setting for normal spectral data (no derivatives). Conversely, maximum p values allow maximum negative contributions (Figure 9 right).

The baseline can be perfectly reproduced for any spectrum using only these two parameter values.

Adjust the baseline primarily by varying the lambda value (either via the slider or by directly typing the desired number in the corresponding textbox) until optimum is reached, ideally as close to linear baselines as possible for vibrational spectroscopic (in particular FTIR spectroscopic) data. Results can be seen in black in the “Selected Spectrum” plot (see 3.2 Selecting a spectrum for display). If there are marked spectra, the “Marked Spectra” plot of the Visualisation Section updates as well, showing the effects of the baseline correction (Mark Classes).

Warning/Information: The sliders can be hard to set exactly. For precise values, use the textboxes. It is especially recommended to use the textbox for lambda values, since it is on a broad logarithmic scale from 1 to 109. Thus, moving the slider can a) result in big jumps in lambda values) and b) result in long, hard to report, hard to remember and hard to reproduce numbers. On the other hand, providing an easier number in the textbox that is close enough in value will produce a virtually identical baseline fit. For example: moving the slider can result in a lambda value of 486425125.9253. In this case, manually providing 500000000 in the textbox can be the preferred alternative, as the baseline will be virtually identical but the number “5x108” is more practical.

It can happen that no optimum baseline can be found, for example due to the presence of negative bands (from e.g. automatic background subtractions resulting in negative CO2 bands in FTIR spectra) or due to the presence of artefacts (e.g. in PCA recombined noise filtered datasets, as in the high wavenumber end in the “demo_image.txt” example file). In this case, try changing the p value or trim the spectral region to eliminate the problematic zone (in case of the “demo_image.txt” file, it is sufficient to set the high edge of the spectral region to “1800” instead of the original “1802” (see 3.3. Spectral Region Adjustment (Cutting)) to eliminate the artefact and produce a good quality baseline).

Figure 9: Asymmetrical Least Squares baseline fitting. Left: overfitted baseline (lambda = 1 (minimum), p = 0.001 (minimum)), middle: underfitted baseline (lambda=109 (maximum), p=0.001 (minimum)), right: demonstrating the effect of high p values (resulting in a baseline that is higher or cuts through the spectrum, producing negative spectral intensity values as a result (lambda=109 (maximum), p=0.999 (maximum)). The example uses the “demo_image.txt” file after cutting the high edge to “1800”.

Figure 10: Optimum baseline (lambda=50 000, p=0.001 (minimum)). The example uses the “demo_image.txt” file after cutting the high edge to “1800”.

Warning/Information: If spectra are to be pre-processed WITHOUT ANY BASELINE correction, use lambda = 1 and p =1.

Warning/Information: p=1 is only allowed when lambda = 1. If the lambda value is higher than 1, the upper limit of p is set to 0.999, to avoid errors in the algorithm.

## 3.5. Normalisation

Normalisation is an optional spectrum standardisation step. This is not required for MCR-ALS, which has a built-in spectral equal length constraint in the App by default (if no normalisation is performed). However, it can still be beneficial if the spectra need to be standardised and exported for further processing by other methods, or simply for visualisation, or demonstration / educational purposes.

To perform normalisation, select the type of normalisation from the “Normalisation” drop-down menu. The default setting is “None”.

Warning/Information: Normalisation overrides the closure constraint for MCR-ALS (see Closure). To keep the Spectrum Equal Length closure constraint, choose “None” for normalisation.

Warning/Information: Normalisation is only performed when the pre-processing parameters need to be updated (such as when the “Save All Pre-Processed”, “Show All Pre-Processed” or “Pre-Processed for MCR-ALS” buttons are clicked in the App, or intensity maps need to be generated (see 3.7. Integration and 3.8. Single Point Intensity Evaluation). However, if there are marked spectra (see Mark Classes), the “Marked Spectra” plot in the Evaluation Panel of the App immediately updates to reflect normalisation changes!

Warning/Information: The “Orig Tot Int” map in the Evaluation Panel ALWAYS shows the original total intensity without normalisation, i.e. the raw input. If, for some reason, a normalised total area map is required (for instance to detect erroneous spectra in the dataset that contains no information, etc.) it can be generated by performing integration over the entire spectral region (see 3.7. Integration). In this case, the Evaluation Panel will label this as an Integral plot, which will be added to the list of Visualised plots in the drop-down list (see Change Plot), without overwriting the original non-normalised total intensity map.

The types of normalisations that can be performed are as follows:

### 3.5.1. Total Area

The area under all bands in the entire spectral region is set to a constant value. In practical terms, each intensity (i.e. the value at each wavenumber) is divided by the total sum of all intensities, i.e. the sum of all intensities is kept constant (Figure 11b).

### 3.5.2. Total Min-Max

The lowest intensity in the entire spectral range is set to 0, and the highest is set to 100. The spectrum is scaled accordingly. Each spectrum is treated this way, independent of one another. That is, the minimum and maximum intensities do not have to be at the same wavenumbers in every spectrum, but every spectrum will have an intensity range 0 – 100 (Figure 11c).

### 3.5.3. Region Area

Similar to the Total Area normalisation, but instead of using the entire spectral range, each spectrum is normalised for the area in a user specified range only. The range is specified by the controls in the “Integral” box in the App (see 3.7. Integration). This option can be used to normalise for the intensity of a certain band, using it as an internal standard (provided that its intensity can be accurately determined) (Figure 11e).

### 3.5.4. Region Min-Max

Similar to the Total Min-Max normalisation, but instead of using the entire spectral range, each spectrum is normalised using a user defined range only. The range is specified by the controls in the “Integral” box in the App (see 3.7. Integration). The minimum intensity in this region is set 0, and the maximum to 100 as in the Total Min-Max normalisation option. This option is similar to the Region Area normalisation above, but scales along the intensity axis, and not for the band area (Figure 11f).

### 3.5.5 Region Max

Similar to the Region Min-Max normalisation, but it only sets the maximum intensity in the region to 100 and sets the minimum intensity to 0, wherever that is in the spectrum (i.e. it does not have to be in the region), and scales the spectra accordingly. The region in which the maximum intensity is set to 100 is specified by the controls in the “Integral” box in the App (see 3.7. Integration).This normalisation option can be preferred over Region Area and Region Min-Max normalisation if the spectral intensity does not decrease to baseline (0) in the desired region due to e.g. overlapping bands (Figure 11d).

### 3.5.6. Point Max

Similar to the Region Min-Max normalisation, but it sets the highest intensity to 100 at a defined point, and not in a range. The point is defined by the “Single Point Intensity” input in the App (see 3.8. Single Point Intensity Evaluation). Since it uses only one point to set the maximum, not a range, it is very sensitive to band shifts. The minimum intensity in the entire spectral range is set 0 (Figure 11g).

### 3.5.7. Offset

Similar to Point Max normalisation, in a sense that it uses a point defined by the Single Point Intensity input in the App to perform the normalisation (see 3.8. Single Point Intensity Evaluation), but instead of setting the intensity to 100 at this point, it sets it to 0 (i.e. baseline). Most importantly, it does NOT scale the spectrum, only shifts (offsets) all intensities (i.e. no multiplication, only addition). Consequently, it is best used on a baseline point (Figure 11i).

Figure 11: Illustrating the effects of normalisation. a: No Normalisation; b: Total Area Normalisation; c: Total Min-Max Normalisation; d: Region Max Normalisation, using the 1550-1640 cm-1 region (aromatic -C=C- band); e: Region Area Normalisation, using the 1550-1640 cm-1 region (aromatic -C=C- band); f: Region Min-Max Normalisation, using the 1550-1640 cm-1 region (aromatic -C=C- band); g: Point Max Normalisation, using the intensity at 1600 cm-1 (aromatic -C=C- band); h: Offset Normalisation, using the intensity at 1600 cm-1 (aromatic -C=C- band); i: Offset Normalisation, using the intensity at 1200 cm-1 (assumed baseline point). The example uses the “demo_image.txt” file, after optimum baseline correction as shown in Figure 10 (spectral region high edge cut to “1800”, asymmetrical least squares baseline lambda=50 000 and p=0.001 (minimum)).

## 3.6. Smoothing

Savitzky-Golay filtering is an optional step that can be applied to smooth the spectra, potentially improving signal-to-noise ratio. The App utilises MATLAB’s built-in “sgolayfilt” function. The smoothing uses two parameters: “Order”, referring to the polynomial order of the fit, and “Frame”. Consult MATLAB’s documentation for details.

Warning/Information: The Frame can only be an odd number, and must ALWAYS be higher than the Order, which must be a positive integer (even or odd). The App will not allow wrong input.

The smoothing is only performed if the “S-G filter” checkbox is ticked. The “Selected Spectrum” plot updates to show the smoothed spectrum in blue. If there are marked spectra, the “Marked Spectrum” plot in the Evaluation Panel of the App also updates automatically.

Generally, the larger the difference between Order and Frame, the more intensive the smoothing is. Often low Order and Frame rates (1 and 5, respectively, for instance) are sufficient for good quality data. Higher polynomial orders can avoid significant broadening, but it is best to experiment with various settings to find the ideal parameters. Always make sure to avoid over-smoothing (Figure 12, right).

Figure 12: The effects of smoothing. Left: correct (mild) smoothing resulting in reduced signal to noise ratio (Order=1, Frame=5). Right: over-smoothing, resulting in loss of information and artefact generation (Order=1, Frame=33). The example uses the “demo_image.txt” file, after optimum baseline correction in Figure 10 (spectral region high edge cut to “1800”, asymmetrical least squares baseline lambda=50 000 and p=0.001 (minimum)).

## 3.7. Integration

The area under the spectral bands can be determined using the controls in the “Integral” box of the Pre-Processing Panel of the App.

Warning/Information: This is a crude integration, with no deconvolution, only the sum of intensities between the low and the high edge of the specified region (see Figure 13).

To perform the integration, simply move the sliders or manually type in the low and high edge of the integral region (preferred, as it is more precise) and click “Evaluate”.

Warning/Information: For large datasets, integration can take longer on slow computers if there has been no pre-processing done prior to clicking on the “Evaluate” button, or if pre-processing parameters have been changed since the last time integrals were evaluated (i.e. new baseline, smoothing, normalisation or cut has been performed). The reason for this is that all spectra must be pre-processed before the integral intensities are evaluated. A status bar shows the progress. Please wait until it finishes.

Naturally, baseline correction, smoothing and normalisation all affect the Integral values (Figures 13 and 14). Thus, if any of them are changed after the integration has been performed, the “Evaluate” button needs to be clicked again to refresh the Integral values. In this case, the entire dataset will be pre-processed again (see the Warning/Information above).

Warning/Information: There is NO information showing whether the integral currently displayed in the Visualisation plot has been calculated with the current pre-processing parameters or not!

The integral area can be shown in the “Selected Spectrum” plot (as a light blue area) by ticking the corresponding “Show in Spectral Plot” checkbox (Figure 13). However, the Visualisation plots are automatically updated whenever the “Evaluate” button is clicked, even if the “Show” checkbox is unticked. The “Visualised” drop-down list is also updated to include the integral plot, using a label starting with “Int @”, followed by the integral region and finished by an abbreviation to denote the kind of normalisation performed (Figure 14). “NN” stands for “No Normalisation”, “TA” for “Total Area”, “TMM” for “Total Min-Max”, “RA” for “Region Area”, “RMM” for “Region Min-Max”, “RM” for “Region Max”, “PM” for “Point Max” and “OFN” for “Offset Normalisation” (see 3.5. Normalisation). The colour map of the plot can be changed by the “Colours” drop-down list in the Evaluation Panel of the App (see Change Colour).

Warning/Information: There can only be one integral evaluated and visualised with the same region and normalisation type (i.e. with the same name). Any new integral over the same region using the same normalisation type (but different smoothing or baseline correction) will automatically overwrite the previous ones without warning. If the integral results are to be kept, they need to be saved before a new integral is evaluated. Integrals and single point intensities do NOT overwrite each other. The original, unprocessed total intensity plot (“Orig Tot Int”) is always kept, irrespective of any integration.

If the entire spectral range is specified for the Integral, it generates a total area map. However, this as an Integral plot, which will be added to the list of Visualised plots in the drop-down list, and will NOT overwrite the original, unprocessed Total Area map (see Warning/Information above), even if no processing is done before integration.

If the spectral region is cut after the integral region was set, there are two possibilities. 1) The cut region still contains the entire integral region. In this case, nothing happens to the integral. 2) The cut region does not contain the entire integral region. In this case, the integral region will be adjusted in the controls to reflect the change, but a new integral is NOT calculated until the “Evaluate” button is clicked.

Warning/Information: Following the resetting of the spectral range to its original values after it has been cut, the Integral controls do not change. Keep this in mind if the total area intensity needs to be determined after reset! In that case, the low and high edges of the integral must be manually reset.

Figure 13: Integration. Left: showing how crudely the integral values are calculated (see the abrupt end of the area at 1640 cm-1). Right: smoothing, baseline correction and normalisation affects the integral value. The example uses the “demo_image.txt” file.

Figure 14: Visualising the integral result. The “Visualised” drop-down list is updated, together with the plot and its title, to show the evaluated integrals. The example uses the “demo_image.txt” file with the same settings and integrals as in Figure 13 above, the top image corresponding to Figure 13 left, and the bottom one corresponding to Figure 13 right.

After evaluating the integral, the corresponding “Save All” button will become active, allowing for saving all the integrals in .mat or .xlsx formats (see Integral and Single Point Intensity Matrices).

Warning/Information: The “Save All” button saves the integrals as they are listed in the Visualisation plot. If there have been changes (in pre-processing or in integration) since they were displayed, click the “Evaluate” button before attempting to save the results.

Warning/Information: The saved integral results cannot be loaded and visualised in App afterwards (note that the button has a black text, not blue).

Warning/Information: If the generated visualisation plots are to be kept as well, not only the intensity values, they need to be saved separately, and individually, by manually changing the Visualisation drop-down menu, hovering over the plot and selecting the save option (see Figure 1).

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file when saving the integral results. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Integrals at the end of an already existing file, such as the raw input data.

Warning/Information: If the results are saved in .xlsx format, the original spectrum identifiers (spectrum names) are retained as well.

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

## 3.8. Single Point Intensity Evaluation

The intensity at any given wavenumber in the spectrum can be evaluated for the entire dataset, using the “Single Point Intensity” edit field of the Pre-Processing Panel of the App. This is a very crude and basic evaluation, just reading out the intensity value at that point in the spectrum (see Figure 15).

Simply enter the wavenumber at which the intensity should be determined in the edit field and click the corresponding “Evaluate” button.

Warning/Information: For large datasets, even this simple intensity evaluation can take a long time on slow computers if there has been no pre-processing done yet, or pre-processing parameters have been changed since the last time single point intensities were evaluated (i.e. new baseline, smoothing, normalisation or cut has been performed). The reason for this is that all spectra must be pre-processed before the intensities are evaluated. A status bar shows the progress. Please wait until it finishes.

Naturally, baseline correction, smoothing and normalisation all affect the Single Point Intensity values (see Figures 15 and 16). Thus, if any of them are changed after the single point intensity evaluation has been performed, the “Evaluate” button needs to be clicked again to refresh the values. In this case, the entire dataset will be pre-processed again (see the Warning/Information above).

Warning/Information: There is NO information showing whether the integral currently displayed in the Visualisation plot has been calculated with the current pre-processing parameters or not!

The position in the spectrum at which the single point intensity is evaluated can only be shown in the “Selected Spectrum” plot (as a vertical magenta line) by ticking the corresponding “Show in Spectral Plot” checkbox (Figure 15). However, the Visualisation plots are automatically updated whenever the “Evaluate” button is clicked, even if the “Show in Spectral Plot” checkbox is unticked. The “Visualised” drop-down list is also updated to include the single intensity plot, using a label starting with “SPInt @”, followed by the wavenumber at which the intensity was evaluated and finished by an abbreviation to denote the kind of normalisation performed (Figure 16). “NN” stands for “No Normalisation”, “TA” for “Total Area”, “TMM” for “Total Min-Max”, “RA” for “Region Area”, “RMM” for “Region Min-Max”, “RM” for “Region Max”, “PM” for “Point Max” and “OFN” for “Offset Normalisation” (see 3.5. Normalisation). The colour map of the plot can be changed by the “Colours” drop-down list in the Evaluation Panel of the App (see Change Colour).

Warning/Information: There can only be one single point intensity evaluated and visualised for the same wavenumber and normalisation type (i.e. with the same name). Any new single point intensity at the same wavenumber using the same normalisation type (but different smoothing or baseline correction) will automatically overwrite the previous ones without warning. If the single point intensity results are to be kept, they need to be saved before a new one is evaluated. Integrals and single point intensities do NOT overwrite each other.

If the spectral region is cut after the single point intensity position was set, there are two possibilities. 1) The cut region still contains this position. In this case, nothing happens to the single point intensity. 2) The cut region does not contain the position set for the single point intensity evaluation. In this case, the single point intensity position will be automatically set to the highest wavenumber of the new (cut) spectral region, but a new evaluation at this position is NOT performed until the “Evaluate” button is clicked.

Warning/Information: Following the resetting of the spectral range to its original values after it has been cut, the single point intensity position does not change.

After evaluating the single point intensity, the corresponding “Save All” button will become active, allowing for saving all the single point intensities in .mat or .xlsx formats (see Integral and Single Point Intensity Matrices).

Warning/Information: The “Save All” button saves the single point intensities as they are listed in the Visualisation plot. If there have been changes (in pre-processing or in integration) since they were displayed, click the “Evaluate” button before attempting to save the results.

Warning/Information: The saved single point intensity results cannot be loaded and visualised in App afterwards (note that the button has a black text, not blue).

Figure 15: Single Point Intensity Evaluation. Left: showing where the intensity values are calculated (see vertical magenta line at 1600 cm-1). Right: smoothing, baseline correction and normalisation affects the intensity value. The example uses the “demo_image.txt” file.

Figure 16: Visualising the Single Point Intensity Evaluation result. The “Visualised” drop-down list is updated, together with the plot and its title. The example uses the “demo_image.txt” file with the same settings and integrals as in Figure 15 above, the top image corresponding to Figure 15 left, and the bottom one corresponding to Figure 15 right.

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file when saving the integral results. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Single Point Intensities at the end of an already existing file, such as the raw input data.

Warning/Information: If the results are saved in .xlsx format, the original spectrum identifiers (spectrum names) are retained as well.

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

Warning/Information: If the generated visualisation plots are to be kept as well, not only the intensity values, they need to be saved separately, and individually. Manually change the Visualisation drop-down menu, hover over the plot and select the save option (see Figure 1).

## 3.9. Saving and Loading Pre-Processing Parameters

To make it easier to process different datasets in identical ways, the pre-processing parameters can be saved and loaded, by using the respective buttons in the App. The parameters that are included in the save / load are listed in the section: Pre-Processing Parameter Matrix.

Warning/Information: If parameters in the loaded matrix fall outside of the boundaries of the current dataset, warning messages will be shown for each of them, and the App will ignore those settings and revert for the default values for those parameters.

## 3.10. Displaying the Pre-Processed Data

The results of the pre-processing are only shown “live” in the App on the spectrum that is displayed in the “Selected Spectrum” plot and on the spectra that are marked in the Evaluation Panel (if there are any. See Mark Classes). To visualise the effect of the pre-processing on all spectra, click on the “Show All Pre-Processed” button. A separate figure window will pop up, displaying ALL spectra in the dataset in a single plot. The new pop-up window has a full menu and icon bar, allowing the change of display parameters (zoom, add legend, etc.) and saving the plot (in all image formats MATLAB supports (see Plots). Note that all spectra are displayed, even if there is more than a 100 (as opposed to the “Original Spectra” plot, which only shows 100 randomly selected ones if there are more than 100 in the dataset).

Warning/Information: Since ALL spectra will be displayed, this can be a slow process on less powerful computers, if pre-processing needs to be performed. A status bar will indicate the progress. Please wait until it completes.

## 3.11. Saving the Pre-Processed Data

The first exit point of the App is the saving of the pre-processed data. It marks the end of the analysis if the App is only used for pre-processing purposes. It is performed by simply clicking the “Save Pre-Processed” button. A dialog window opens where the filename and the folder can be specified. The App automatically suggests a filename, based on the original, adding the X and Y dimensions at the start and appending the word “Raw, “Cut” or “Processed” in the end, depending on what processing was done. “Raw” means the data was not processed in any ways, and the App is only used to convert its format (from .txt to .mat, for example).  “Cut” means the spectral region was trimmed, but nothing else has been done. In any other cases, the label is “Processed”. The saved data is in a format that can be directly loaded by the App without modification later (hence the blue text on the button).

Warning/Information: Irrespective of the initial input format, data can be saved as a .mat file (for the format of the resulting .mat file, see Cut and Pre-Processed Spectra) or as an .xlsx file. Thus, image .txt inputs that contained the X and Y coordinates will lose this information. This is why the App automatically suggests to append the dimensions to the filename in the XXX_YYY number format. Saving in .txt format is not possible.

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Processed spectra at the end of an already existing file, such as the raw input data.

Warning/Information: If the results are saved in .xlsx format, the Pre-Processing Parameters are automatically included as well, in a separate worksheet.

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

## 3.12. Passing the Data for MCR-ALS

The data can be passed for MCR-ALS in two ways: unprocessed or pre-processed, using the respective buttons in the App. Passing the data without processing can be preferred if the data has already been processed before (for instance by the App, saved (3.11. Saving the Pre-Processed Data) and now re-loaded) or if the unprocessed data is used for comparison / reference, etc. Otherwise pre-processing (especially 3.4. Asymmetrical Least Squares Baseline Correction) is generally recommended.

Warning/Information: If pre-processing needs to be performed, it can be slow for large datasets and slow computers. A status bar will indicate the progress. Please wait until it completes. If, however, pre-processing has already been performed (e.g. for 3.7. Integration) and no pre-processing parameters have been changed since then, passing the data for MCR-ALS is fast.

Once the data is passed for MCR-ALS, the MCR-ALS Panel of the App starts to populate, including the “Singular Value Decomposition” plot, the “Eigen Value” drop-down list, the “# Components” edit field, the MCR-ALS “Direction” drop-down list, the “Noise (%)” edit field, the “Purest” list box and the “Initial Estimates” plot. The “Load Input” button and the “Show Purest” checkbox are also activated (Figure 17).

Warning/Information: If there have already been MCR-ALS results in the MCR-ALS part of the App, these will be permanently erased from the App without a warning! Variables in the MATLAB Workspace will remain, however (see Perform MCR-ALS).

Figure 17: Passing the data for MCR-ALS. Additional elements of the App are activated and plots populated. The example uses the “demo_image.txt” file.

# MCR-ALS

Multivariate Curve Resolution – Alternating Least Squares (MCR-ALS) is an unmixing technique based on the assumption that any spectrum can be described as the linear combination of a set of “pure” component contributions (plus some noise). “Pure” components ideally mean pure chemical compounds, but such resolution is not always possible. Thus, in practice, “pure” means “as clearly resolved as possible”. As an iterative technique, the level of resolution depends on the iteration parameters (see Number of Iterations and Convergence Criterion), and to a large extent on the initial estimates from where the iterations start (to avoid local minima and find absolute minimum) (see Initial Estimates). In addition, resolution can be aided (the number of potentially correct solutions reduced) by applying various constraints (see Constraints). Naturally, the input data is of vital importance, thus pre-processing significantly affects the results.

Warning/Information: There is NO INDICATOR telling whether the current Pre-Processing values have been implemented or not yet. For instance, if spectra have been pre-processed with a set of parameters, and MCR-ALS analysis has been performed, the optimised spectra and concentration plots will be populated. If the user now changes any of the pre-processing parameters, the MCR-ALS results remain in the App display but they are still referring to the results obtained by the previous pre-processing, until a new MCR-ALS is done. This is not a bug, it is a deliberate design feature.

Although the App is a significantly improved version of the GUI published in Nature Protocols in 2015 (http://www.nature.com/nprot/journal/v10/n2/abs/nprot.2015.008.html), it still does NOT include several extremely powerful and useful MCR-ALS tools. For instance, it does not utilise image information to aid the resolution (such as local rank constraints), and it cannot simultaneously handle multimethod and/or multiset data. For a more versatile MCR-ALS package and more information about MCR-ALS, visit www.mcrals.info. General limitations of iterative solutions are inherent to the method and thus remain in this App as well.

## Number of Components

The first step in performing MCR-ALS is to determine the number of pure components that are required to describe the dataset. This can be done based on a priori knowledge of the system or by analysing the dataset using singular value decomposition.

If the number of components is known, type the number directly in the “# Components” box in the MCR-ALS Panel of the App. Alternatively, use the “Eigen Value” drop-down list to see the numerical results of the singular value decomposition, and select the one after which a large drop in Eigen Values is observed, or when the Eigen Values approximate 0 (this can be observed in the “SVD” plot of the MCR-ALS Panel of the App) (Figure 18).

Warning/Information: Double-clicking any element of the “Purest” listbox will remove it and adjust the number of components accordingly. This only works until only 1 element remains in the “Purest” listbox. The last remaining element cannot be removed (i.e. the “Purest” listbox cannot become empty). If this element needs to be removed, a new one must first be added.

After providing the number of components (either through the “Eigen Values” drop-down list or via the “# Components” textbox), the SVD plot zooms in to show only the number of components + 5 values (to have a better overview), the Initial Estimates plot and the “Purest” spectra listbox updates (see Automatic Estimation and Show Purest) and the “Show PCA” button becomes active (Figure 17).

Figure 18. Determining the number of components by singular value decomposition. Left: the “demo_image.txt” data (pre-processed with the parameters shown in Figure 17) displays a gradual decrease with no abrupt drop in Eigen Values. The number of components can reasonably be assumed to be 3, 4 or 5 (6 and 7 are already close to 0 on the SVD plot) and it is recommended to test MCR-ALS with all those possibilities. Right: the unprocessed “demo_indep_spectra.xlsx” dataset has a clear drop in Eigen Values after the 3rd component. Thus, this data is most likely to have 3 components.

Warning/Information: Using the “Eigen value” dropdown or the “#Components” edit field will remove ALL externally loaded Initial Estimates (see Load Input) and ALL Initial Estimates obtained via marked pixels (see Use Marked Spectra) without warning!

Warning/Information: Technically it is possible to select (close to) as many pure components as there are spectra in the dataset, but this is practically meaningless and can cause errors / warnings in the MCR-ALS algorithm (overfitting). Try to keep the number of components as low as possible. For similar reasons, the maximum number of singular values is limited to 30. This is a completely arbitrary number, and can easily be changed in the source code of the App without affecting any other parts of the script.

Warning/Information: The MCR-ALS process requires more than 1 component. If only 1 component is selected, the “Perform MCR-ALS” button is deactivated automatically.

Warning/Information: Determining the number of components by singular values can be less than straightforward at times, since the values may not drop sharply at a certain point but gradually decrease (Figure 18). In this case, try the following: a) test different numbers of components, perform MCR-ALS and observe the results to decide which is the optimum; b) use any a priori knowledge / reasoning to motivate the optimal number of components (try to keep to as low numbers as possible); c) adjust pre-processing parameters and see if this helps determining the number of components.

## Show PCA

After the number of Components have been set (Number of Components), principal component analysis (PCA) of the data becomes possible, using as many principal components (PCs) for the reconstruction of the dataset as the number of Components. Click on the “Show PCA” button to perform the PCA analysis and to visualise the results in a separate, pop-up window. This window contains 4 plots: 1) the Initial Pure Spectra Estimates, 2) the unconstrained concentration profiles (using a single iteration least squares fit), compared to the 3) PCA Loadings and 4) Scores, respectively, prior to MCR-ALS (Figure 19). This could be especially interesting when initial estimates are not determined automatically but are supplied in another way (manually loaded, selected from marked pixels or from reference spectra (see Initial Estimates)).

Figure 19. PCA reconstruction results visualised in a separate pop-up window after clicking the “Show PCA” button in the App. The example uses un-processed “demo_indep_spectra.xlsx” dataset with 3 Components

## Initial Estimates

The initial estimates are very important to the success of the MCR-ALS resolution. The App therefore allows for different methods of providing them.

The direction of the MCR-ALS procedure (see Direction) determines the kind of input required: “Spectrum” direction (default in the App) requires spectra as initial estimate input, while “Concentration” direction naturally requires concentration values as input. The examples below only use the “Spectrum” direction for illustration (i.e. in Figures) because these are easier to follow (less abstract) and some options are only viable for those (such as Use Marked Spectra and Use Reference Spectra), but notes are given for “Concentration” direction as well.

Potential overlaps with constraints are listed as well.

### Automatic Estimation and Show Purest

The default option in the App is the automatic estimation of the initial values, using a SIMPLISMA-based algorithm (see www.mcrals.info for more information). This method works both in the Spectrum and the Concentration directions, and is completely automatic. It is performed as soon as the data is passed for MCR-ALS from the Pre-Processing Panel of the App (3.12. Passing the Data for MCR-ALS), and Initial Estimates are updated automatically whenever the number of Components is changed (see Number of Components), including updating the Initial Estimates plot and the estimation of the purest spectrum (or wavenumber if Concentration direction is used) (see Figure 20).

The only control the user have is the “Noise” textbox, where the level of allowed noise can be set (in percentages) for the Initial Estimation procedure. The default value of 10 % Noise is usually sufficient, and small changes in the Noise percentage usually have no or minimal effect on the initial estimates on normal vibrational spectral datasets. This can be easily tested by setting different noise levels and observing the changes in the Initial Estimates plot and selecting the best result.

The “Show Purest” checkbox is activated, allowing for displaying which spectrum in the dataset is estimated to be the purest for each Component. When this checkbox is ticked, white diamonds with black outline appear in the active map plot in the Evaluation Panel of the App (Figure 20). Naturally, if Concentration direction is selected (see Direction) the purest spectra cannot be shown in the active map plot, even if the checkbox is ticked.

Warning/Information: The Initial Estimates are in Component order in the “Purest” listbox (i.e. the first Component is listed as first), as long as no external input was made (from e.g. Reference Spectra (Use References as Initials), or from loading an input file (Load Input)). However, the Markers on the visualisation plots have no labels to indicate which Component they belong to (see Figure 20 for an example).

Warning/Information: The Markers remain even if the Visualised plot is changed using the “Visualised” drop-down list (see Change Plot) or a different colour coding is selected via the “Colours” drop-down list (see Change Colour), as long as the checkbox is ticked.

Warning/Information: The position of the Markers automatically updates when the image is refolded / reset (see Change Dimensions (Refold and Transpose)).

Warning/Information: The Markers automatically update when the number of Components is altered (see Number of Components).

Warning/Information: Markers do NOT appear for externally loaded initial estimates (from e.g. Reference Spectra (Use References as Initials), or from loading an input file (Load Input))

Warning/Information: If the Markers are visible in the Evaluation Panel of the App, they will be visible in the Comp plots as well (Show Component Plots and Save Comp Plots).

Warning/Information: If Concentration direction was selected, the Show Component Plots and Save Comp Plots options will show a dashed vertical line for the purest wavenumbers in the spectral plots, instead of the white diamonds of the purest spectra in the concentration maps.

Warning/Information: If the legend obscures the “Initial Estimates” plot, it can be moved to a different position within the plot, by left-clicking and dragging.

Figure 20. Automatic determination of Initial Estimates and marking the purest spectra on the Visualisation plot. The example uses the “demo_image.txt” file, the exact same way as in Figure 18 left. The 1st Component is shown in blue in the “Initial Estimates” plot and the purest spectrum belonging to this Component is estimated to be nr. 103 (1st value the “Purest” listbox). The 2nd Component is sown in red in the “Initial Estimates” plot and the purest spectrum belonging to this Component is estimated to be nr. 10 (2nd value the “Purest” listbox). The 3rd Component is shown in yellow in the “Initial Estimates” plot and the purest spectrum belonging to this Component is estimated to be nr. 72 (3rd value the “Purest” listbox). However, there is no label on the Markers in the Visualisation plot, and their colours do not represent the Initial Estimates plot colours either (they are all white diamonds with black outlines). Based on the spectrum number, it can be deduced, however, that the bottom left marker must be spectrum 103 (the highest number), and thus Component 1, the topmost marker (in the first row of the image) must be spectrum 10 (the lowest number) and thus Component 2, while the marker most to the right must be spectrum 72 and thus Component 3.

### Load Input

Input data can be manually loaded, using the “Load Input” button in the MCR-ALS Panel of the App, to complement / replace the automatically determined Initial Estimates.

This method works in both Concentration and Spectrum directions!

The Inputs to be loaded must be in an Excel .xlsx file that meets the formatting requirements as described in the section “Initial Estimate Input” and must match the direction of the Initial Estimates (spectrum or concentration). The Loaded Initial Estimates are added to the already listed Initial Estimates and displayed in the “Initial Estimates” plot as well as in the “Purest” listbox, using the names supplied in the input .xlsx file (Figure 21). The App automatically checks for compatibility and will not allow incompatible input to be loaded. If any initial estimate was loaded manually, the SVD plot will show a warning message (since the computed singular values are based on the dataset itself, and do not include the externally loaded input). Likewise, the “Eigen-values” drop-down will have red text and the “# Components” edit field background will change to yellow, indicating that the number of components is not based on the Eigen-values.

Warning/Information: The “Eigen values” drop-down list and the “# Components” edit field will automatically update to the total number of components, even if the components include externally loaded ones.

If some of the Initial Estimates need to be removed after loading (either from among the automatically determined ones or from the newly loaded ones), double-click its descriptor in the “Purest” listbox.

Warning/Information: Double-clicking any element of the “Purest” listbox will remove it and adjust the number of components accordingly. This only works until only 1 element remains in the “Purest” listbox. The last remaining element cannot be removed (i.e. the “Purest” listbox cannot become empty). If this element needs to be removed, a new one must first be added. If there no externally loaded references remain in the “Purest” listbox, the warning message is removed from the SVD plot, the “Eigen values” drop-down reverts to black and the “# Components” edit field background reverts to white.

Warning/Information: Naturally, the loaded Initial Estimates cannot have Markers in the Visualised plot, as they do not feature in the original dataset plotted there (Figure 21).

Warning/Information: If the legend obscures the “Initial Estimates” plot, it can be moved to a different position within the plot, by left-clicking and dragging.

Warning/Information: Equality Constraint vs Load Input: If a certain compound is known / suspected to be a pure component in the dataset, it can be supplied either as an initial estimate or as an equality constraint for the spectral direction. There is a difference in the mechanism how this can practically affect the results. An initial estimate does NOT pose ANY constraints at all during the modelling, it only makes sure that the iteration starts with this value. From that on, the model can take any direction and the spectral profile can change freely, without restrictions (apart from non-negativity). In other words, this way of supplying a spectral profile only ensures that the optimization starts close to the expected final result (i.e. close to the suspected absolute minimum, avoiding falling into potential local minima, or lowering the number of iterations the model requires to reach convergence). On the other hand, if a spectral profile is supplied as a constraint, it poses a more rigid limitation for how the model can evolve. With an “exact” constraint applied, for instance, the component spectral profile is NOT allowed to change at all. This can be useful when a known compound with a known (and exact) spectral profile needs to be tracked in a large dataset, for example. Especially if the selected compound is present only in a few spectra and in low amounts (i.e. its contribution to the total variation is low) The same is true in the concentration direction, with concentration profiles loaded as initial estimates vs supplied as equality constraint.

Figure 21. Load Input manually for Initial Estimates. The example uses the “demo_image.txt” file exactly as in Figure 20, with the manually loaded auxiliary data “InitEstimates_spectra.xlsx”. This auxiliary data file contains the spectral profiles “est1”, “est2” and “est3” (as seen in the “Purest” listbox). Note that these names are NOT listed in the Initial Estimates plot legend, which only contains the colours and the list numbers. It can be deduced, however, that nr. 6 is “est3”, since that is listed as the 6th in the “Purest” listbox. There are no Markers in the Visualisation plot for the newly loaded Initial Estimates. Note also the yellow warning text on the SVD plot, the red text on the Eigen values drop-down and the yellow background in the # Components edit field.

### Use Marked Spectra

Initial Estimates can be manually specified from the current dataset to complement / replace the automatically determined Initial Estimates. These manually specified Initial Estimates need to be first marked in the dataset as belonging to any of the 6 classes possible to specify (class 1 – 6, black, red, blue, green, magenta and cyan, see Mark Classes).

This method ONLY works in the Spectrum direction!

If some of the Initial Estimates need to be removed after loading (either from among the automatically determined ones or from among the ones imported by the “Use Marked” button), double-click their descriptor (spectrum number) in the “Purest” listbox.

Warning/Information: Marked spectra are added to the end of the “Purest” listbox and to the “Initial Estimates” plot in the order they appear in the “Marked” textbox (Figure 22). If they were coloured (MATLAB 2023a and later) they will lose their colouring in the “Purest” listbox. If they already feature anywhere in the “Purest” listbox, they will NOT be added again. In case one of the added Marked spectra is deleted and later re-added, it ends up at the end of the list again. The rest will keep their positions.

Warning/Information: If the “Show Purest” checkbox is ticked, the white diamond markers can completely cover the coloured marker squares as they are always on the top layer of the plot. Untick the checkbox to make those markers visible.

Warning/Information: Double-clicking any element of the “Purest” listbox will remove it and adjust the number of components accordingly. This also removes its diamond marker from the plots, but it does NOT remove it from the Marked spectra. Removing elements from the “Purest” listbox works until only 1 element remains. The last remaining element cannot be removed (i.e. the “Purest” listbox cannot become empty). If this element needs to be removed, a new one must first be added.

Warning/Information: If the legend obscures the “Initial Estimates” plot, it can be moved to a different position within the plot, by left-clicking and dragging.

Warning/Information: The “Initial Estimates” plot can be very misleading at times when Marked Spectra are used as input. This is especially the case if the data was passed on to MCR-ALS and then pre-processing parameters were adjusted. In this case, the Marked Spectra are automatically updated but any Initial Estimates that were there already will not be updated. So adding Marked Spectra in this case will add spectra processed differently than the previous spectra. This is NOT recommended! If the dataset has been re-processed, pass it to MCR-ALS again and add the Marked spectra again.

Warning/Information: The Eigen Values drop-down, # Components edit field and SVD plots will NOT show warnings, since the added spectra are part of the dataset. However, they are likely still erroneous, since the Marked spectra are selected manually and NOT by their Eigen values.

Warning/Information: Equality Constraint vs Marked Spectra: If a certain compound is known / suspected to be a pure component in the dataset (a pure pixel or a pure spectrum in a series), it can be supplied either as an initial estimate by first marking in and then passing it on within the App, or as an equality constraint for the spectral direction. There is a difference in the mechanism how this can practically affect the results. An initial estimate does NOT pose ANY constraints at all during the modelling, it only makes sure that the iteration starts with this value. From that on, the model can take any direction and the spectral profile can change freely, without restrictions (apart from non-negativity). In other words, this way of supplying a spectral profile only ensures that the optimization starts close to the expected final result (i.e. close to the suspected absolute minimum, avoiding falling into potential local minima, or lowering the number of iterations the model requires to reach convergence). On the other hand, if a spectral profile is supplied as a constraint, it poses a more rigid limitation for how the model can evolve. With an “exact” constraint applied, for instance, the component spectral profile is NOT allowed to change at all. This can be useful when a known compound with a known (and exact) spectral profile needs to be tracked in the dataset. The same is true in the concentration direction, with concentration profiles loaded as initial estimates vs supplied as equality constraint.

Figure 22. Use Marked Spectra for Initial Estimates. The example uses the “demo_image.txt” file exactly as in Figure 20, with manually marking spectra 3 and 56 as Class 2 (red) and spectrum 38 as Class 3 (blue), as seen in the “Marked” listbox. The “Marked Spectra” plot shows the spectra in their respective colours as well. Note that the Marked Spectra are added after the automatically determined ones (spectra 103, 10, 72), in the order they appear in the “Marked” listbox, but without their colours. Note that the SVD plot, the “Eigen value” drop-down list and the “# Components” edit box does not show warning messages either, though the Eigen values are erroneous for this set of Initial Estimates.

### Use Reference Spectra

The loaded Reference Spectra can be passed to the MCR-ALS part of the App as Initial Estimates to complement / replace the automatically determined Initial Estimates. First, load the Reference Spectra (see Load Reference Spectra), then click on the “Use Refs as Initials” button. Note that his button is located in the “Reference Spectra Match” section in the Evaluation Panel of the App (Figure 23), not in the MCR-ALS Panel! All Reference Spectra will be added to the list of already existing Initial Estimates, and also displayed in the “Initial Estimates” plot as well as in the “Purest” listbox, using the reference names as they appear in the legend of the Reference Spectra plot (Figure 23). The App checks for duplicates and only adds those references that do not already feature in “Purest” listbox.

This method ONLY works in the Spectrum direction.

If references are used as initial estimates, the SVD plot will show a warning message (since the computed singular values are based on the dataset itself, and do not include the externally loaded references). Likewise, the “Eigen-values” drop-down will have red text and the “# Components” edit field background will change to yellow, indicating that the number of components is not based on the Eigen-values.

Warning/Information: The “Eigen values” drop-down list and the “# Components” edit field will automatically update to the total number of components, even if the components include externally loaded references. If some of the Initial Estimates need to be removed (either from among the automatically determined ones or from the newly added ones from the references), double-click its descriptor in the “Purest” listbox.

Warning/Information: Double-clicking any element of the “Purest” listbox will remove it and adjust the number of components accordingly. This only works until only 1 element remains in the “Purest” listbox. The last remaining element cannot be removed (i.e. the “Purest” listbox cannot become empty). If this element needs to be removed, a new one must first be added.

Warning/Information: Naturally, the Initial Estimates from Reference Spectra cannot have Markers in the Visualised plot, as they do not feature in the original dataset plotted there.

Warning/Information: If the legend obscures the “Initial Estimates” plot, it can be moved to a different position within the plot, by left-clicking and dragging. The Initial Estimates plot can look strange, since the Reference Spectra may have very different intensities than spectra in the dataset.

Figure 23. Use Reference Spectra for Initial Estimates. The example uses the “demo_image.txt” file exactly as in Figure 20, with the manually loaded auxiliary data “demo_references.xlsx”. This auxiliary data file contains the reference spectra “cellulose”, “lignin” and “pectin” (as seen in the “Purest” listbox and in the legend of the Reference Spectra plot). Note that these names are NOT listed in the Initial Estimates plot legend, which only contains the colours and the list numbers. It can be deduced, however, that nr. 6 is “pectin”, since that is listed as the 6th in the “Purest” listbox. There are no Markers in the Visualisation plot for these Initial Estimates either, since the Reference Spectra are not part of the dataset. The Reference Spectra plot shows the spectra in the default MATLAB colour order, which is conflicting the colouring of the “Initial Estimates” plot. Reference Spectra used for Initial Estimates do NOT keep their colours, but will be assigned a new colour depending on their position in the “Purest” listbox (For example, cellulose is blue in the Reference Spectra plot, but it is violet in the Initial Estimates plot, as it has position 4 in the “Purest” listbox, i.e. Component 4). Note also the yellow warning text on the SVD plot, the red text on the Eigen values drop-down and the yellow background in the # Components edit field.

Warning/Information: Equality Constraint vs Load Input: If a certain compound is known / suspected to be a pure component in the dataset, it can be supplied either as an initial estimate via loading it as a reference, or set its equality constraint for the spectral direction if found as a pure estimate via the Eigen values. There is a difference in the mechanism how this can practically affect the results. An initial estimate does NOT pose ANY constraints at all during the modelling, it only makes sure that the iteration starts with this value. From that on, the model can take any direction and the spectral profile can change freely, without restrictions (apart from non-negativity). In other words, this way of supplying a spectral profile only ensures that the optimization starts close to the expected final result (i.e. close to the suspected absolute minimum, avoiding falling into potential local minima, or lowering the number of iterations the model requires to reach convergence). On the other hand, if a spectral profile is supplied as a constraint, it poses a more rigid limitation for how the model can evolve. With an “exact” constraint applied, for instance, the component spectral profile is NOT allowed to change at all. This can be useful when a known compound with a known (and exact) spectral profile needs to be tracked in the dataset. The same is true in the concentration direction, with concentration profiles loaded as initial estimates vs supplied as equality constraint.

## Direction

Use the “Direction” drop-down list in the MCR-ALS part of the App to specify the direction in which the Initial Estimates are determined and the alternating least squares algorithm starts.

With high enough number of iterations and correct convergence limits (see Number of Iterations and Convergence Criterion), the direction of the iterations should have minimal impact on the MCR-ALS results. However, setting the direction to either “Spectrum” (default) or “Concentration” can be helpful in evaluating the quality of the data already before MCR-ALS modelling takes place, simply by inspecting the Initial Estimates. For example, if “Spectrum” direction is selected, the initial spectral profiles can be informative about the compounds the model is estimating to be present in the dataset, and changing the number of components can reveal further information about minor components / noise levels in the data. Similarly, if certain concentration profiles are known / expected (e.g. from monitoring a reaction and knowing at which spectrum a reactant was added, etc), “Concentration” direction can provide a very quick quality assessment already at the Initial Estimates plot.

Warning/Information: Some Initial Estimate inputs are only possible with “Spectrum” direction selected (see Initial Estimates).

Warning/Information: Changing directions will clear the “Purest” listbox and remove all externally loaded input without warning.

## Constraints

Constraints can dramatically alter the result of the MCR-ALS process and can be a valuable tool in reaching optimum resolution. Thus, using the correct constraints constitutes the “art” of MCR-ALS and can make or break a model. However, while correct constraints can salvage something even from vastly suboptimal datasets, ultimately it is the input data quality that determines the success of MCR-ALS. (Conversely, incorrect constraints can easily distort MCR-ALS results even from perfectly good datasets.)

Constraints can be mild (allowing a certain degree of flexibility) or hard (when the rules they set must be strictly followed). Generally, hard constraints should only be used when the data is certain to fulfil their prerequisites, in order to minimise the chance of artefacts / bias in the results.

The present App only contains constraints that have chemical / spectroscopic foundations (e.g. non-negativity), not only mathematical (local rank constraints, etc).

Warning/Information: Constraints need to be applied with care as they have profound effects on the outcome of the model and can constitute to serious bias. Improper constraints can be easy to discover if the MCR-ALS modelling gives any kind of erroneous result or MATLAB shows error or warning messages in the Command Window (such as rank deficiencies, NaN values instead of numbers in the results, etc), but they can be hard to spot if the model appears to work correctly computationally. As a general rule, MCR-ALS models with the minimum necessary constraints are preferred, and constraints should preferably be based on established features of the dataset.

Warning/Information: Constraints can be set independently from each other and multiple constraints can be applied at the same time (for the same or for different components). The more constraints applied, the harder the limitations become.

### Non-Negativity

This is a default constraint in both the Spectrum and the Concentration profiles, hard-coded in the App with no user control. It is implemented as a mild constraint, using a fast non-negative least squares algorithm, as opposed to having a hard non-negativity constraint, where negative values would simply be force-replaced by zeros, without fitting.

The present non-negativity constraints are built-in and cannot be removed or modified in the App without extensive modification of the source code. Thus, the App is not well suited for datasets that inherently contain negativity (e.g. EPR spectra), although this restriction is not a general MCR-ALS feature otherwise.

### Equality

Equality constraints can be set for one, several or all components, independently for concentrations and spectra or both. The default setting is “None”, i.e. no equality constraint applied for either concentration or spectra.

Equality constraints allow the user to fix values in the concentration or spectral profiles, such as the spectrum of a known compound present in the dataset, and/or its concentration in certain spectra / pixels of the dataset (including even 0 concentrations, i.e. the absence of a compound. For instance, the starting material can be fixed at 100% concentration (i.e. being pure) in the first few spectrum of a reaction dataset before the reactant is added, while the concentration of both the reactant and the product(s) can be fixed at 0% in the same spectra).

The method for equality constraints can be set using the drop-down list in the “Equality” box of the MCR-ALS Panel of the App. The default option is “None” (i.e. no equality constraint), but it can be changed to “Exact” (i.e. the constraint values must be met exactly) or “Maximum” (i.e. the constraint values cannot be exceeded, but they are allowed to be lowered). For example, if Component 1 concentration is set to be 80% in a certain spectrum of the dataset, it will remain exactly at 80% in that spectrum during MCR-ALS modelling with the “Exact” method, while it can obtain any value between 0 – 80% during MCR-ALS modelling with the “Maximum” method. (0 is the minimum value it can obtain, due to the fixed non-negativity constraints. Thus, if a concentration is set to 0 in a spectrum, “Exact” and “Maximum” type of equality constraints will have the same effect.)

Warning/Information: Setting “Exact” for spectral equality constraint is a very hard constraint, since it will not consider noise or small shifts in the spectra. While “maximum” can be somewhat milder, it is still much more rigid in the spectral direction than it is in the concentration direction, since it still does not cope well with shifting peaks, or with varying total intensities. Be wary of using spectral equality constraints and make sure the spectral profiles used for constraining are of good quality and matching the dataset.

The method for the equality constraint can only be selected after the respective matrix has been loaded by the “Load Matrix”. If there is no matrix loaded, the constraint drop-down will revert to “None”.

Warning/Information: There is no way of adjusting the equality constraint interactively in the App. Constraints need to be loaded from an already existing .mat file, which must meet the formatting criteria required by the App (see Equality Constraint Matrices and the included “csel.mat” and “ssel.mat” auxiliary data files). The App will show an error message if the dimensions of the loaded matrix do not match (e.g. the number of components is wrong, the number of spectral variables is wrong (spectra cut to different length), etc). In this case, no matrix is loaded and the constraint drop-down value is automatically set to “None”.

Warning/Information: If the number of components changes, all loaded matrices are automatically cleared without warning, and the constraint drop-down value reverts to “None”.

Warning/Information: If the data is passed again for MCR-ALS (e.g. after new pre-processing, but even without processing, see 3.12. Passing the Data for MCR-ALS), all loaded matrices are automatically cleared without warning, and the constraint drop-down value reverts to “None”.

Warning/Information: Once an Equality Constraint Matrix is loaded, it is kept until it is cleared by either changing the number of components or passing the data for MCR-ALS again.

Warning/Information: It is possible to set BOTH spectral and concentration equality constraint, independently from each other. They do not need to concern the same component(s) either, although that is also possible. The more constraints applied, the harder the limitations become.

Two equality constraint matrices are supplied (ssel.mat and csel.mat) as auxiliary data, matching the demo_image.txt file dimension for 3 components, provided that the spectral range of the dataset is cut to 1800 cm-1 on the high edge during pre-processing (see Figure 20). Both of these matrices are extreme (and completely fabricated, i.e. they have nothing to do with the actual dataset apart from matching its dimensions), for pedagogical reasons: the impact of the equality constraint can be easily visualised with such drastic (and unfitting) constraints magnifying their effects.

The csel.mat matrix constrains Component1 to have a concentration of 1 in the first 3 spectra (pixels) of the dataset, and 0 in all other spectra (pixels). In other words, it assumes Component1 to be present only in the first 3 spectra (pixels), and nowhere else, both in case of “Exact” and “Maximum” types of concentration constraining. (The difference between the “Exact” and “Maximum” types can be visualised easily by changing the type of normalisation during Pre-Processing, between “None” and “Tot Area”. This will change the intensity values to be outside or within the range of the csel matrix, respectively). Component 2 is unconstrained in the csel matrix except for the last spectrum (pixel) of the dataset, where its concentration is set to 1 (Figure 24).

Figure 24. a, b, c, and d, in order from left to right. The effect of Concentration Equality Constraints. The example uses the “demo_image.txt” file exactly as in Figure 20 for a and b, and with Total Area Normalisation for c and d. 3 components are selected for MCR-ALS, using automatic initial estimate determination, spectral direction and 10% noise. Convergence limit is set to 1 for speed, and maximum number of iterations is kept at the default 50. Results are identical without normalisation (c.f. a and b) since the constraints are numerically outside the range of the dataset (see the scale on the Y-axis of the Optimised Concentrations Plot). For visualisation purposes, either the csel matrix values need to be adjusted, or the spectra scaled, by e.g. total area normalisation. This highlights the effect of the concentration equality constraint and the differences between “equal” (a hard constraint) and “maximum” (a milder constraint), c.f. c and d.

The ssel.mat matrix constrains Component1 to only contain two bands around 1600 and 1650 cm-1, and 0 values (baseline) everywhere else. The other 2 components are unconstrained and thus can have any spectral profiles during MCR-ALS modelling. Since both “Exact” and “Maximum” are hard constrains for spectral profiles, visualising the differences between these two methods is difficult in the spectrum direction. Slight differences can be noticed when “Point Max” normalisation is used at 1600 cm-1 and MCR-ALS is performed with a Convergence Criterion of 1 (Convergence is reached in Iteration 15 using the “Maximum” method, with 98.3294% variation explained and 10.663% lack of fit. Using the “Exact” method, convergence is reached in 18 iterations only, with slightly worse results: 98.1688% variation explained and 11.3927% lack of fit). The resolved spectral and concentration profiles are very similar, however, due to the severity of the constraint applied (Figure 25).

Figure 25. a, b, c, and d, in order from left to right. The effect of Spectrum Equality Constraints. The example uses the “demo_image.txt” file exactly as in Figure 20 for a and b, and with Point Max Normalisation at 1600 cm-1 for c and d. 3 components are selected for MCR-ALS, using automatic initial estimate determination, spectral direction and 10% noise. Convergence limit is set to 1 for speed, and maximum number of iterations is kept at the default 50. Results are almost identical without normalisation (c.f. a and b) and somewhat more differing with normalisation (c.f. c and d). The effect of normalisation is drastic in both the concentration and spectral profiles.

### Unimodality

Unimodality constraints can be set independently for concentration and spectral profiles, by first selecting the mode unimodality should be applied for each, in their respective drop-down list. “None” means no unimodality constraint is applied. “Vertical” and “Horizontal” modes mean that secondary maxima are cut either vertically or horizontally, respectively, during modelling. “Average” mode means that secondary maxima are adjusted by averages (similarly to least squares fitting algorithms). While this is the smoothest application of the constraint, it is also by far the slowest, due to nested loops inside conditional statements.

Warning/Information: Choosing “Average” with large number of variables (i.e. many spectra if concentration unimodality is set to “Average”, or many spectral points (high spectral resolution and/or wide spectral region) if spectra unimodality is set to “Average”) can be extremely slow.

By default, all components are listed to be constrained and the tolerance is set to “1” (Figure 24), but the constraint is set to “None” by default.

If only a certain number of components need to be constrained, their number needs to be entered in the respective “Components” edit field, separated by commas. No comma should be present after the last number. For instance, if only components 1 and 3 need to constrained, enter “1, 3” or “1,3” (without spaces) in the edit field. Once the numbers are entered, hit “Return” / “Enter” to approve the list.

Warning/Information: The component list for concentration and spectrum unimodality are independent. Thus, any component may be constrained by only in concentration or spectral profiles, or both (or neither).

Warning/Information: The App does not allow for incorrect input such as values that are not numbers, more items than that the number of components, higher numbers than the number of components. In this case, a warning message is shown, and the list reverts to its default values.

Warning/Information: If any component is entered multiple times (e.g. 1,1,2 or 1,2,1), replicates will be removed automatically.

The tolerance levels for the unimodality constraints can be set in the “Tolerance” edit field. The tolerance level softens the unimodality constraint by allowing for a certain level of local departures (i.e. “noise”). For example, 1.3 means a maximum of 30% local departure is allowed. Thus, on the decreasing slopes of the main peak, a particular point can have an increased value that corresponds to maximum 30% of the previous value before the unimodality constraint is applied (i.e. non-monotonous decrease or noise). Values between 1.0 (hardest constraint with no local departures allowed, only monotonous slopes) and 1.1 (10% departure allowed) are usual in systems with low to medium noise levels.

Warning/Information: It is NOT possible to set different (individual) tolerance levels to each component, but it is possible to set different tolerance levels for the spectrum and concentration unimodality constraints.

Warning/Information: As a general rule, unimodality constraints should NOT be applied to image datasets and independent spectra. Moreover, unimodality constraints are seldom useful for spectral profiles, as they more or less assume a single maximum (i.e. a single peak) in the spectrum. They are most useful for the concentration profiles in a reaction series (Figure 26).

Figure 26. a, b, c, and d, in order from left to right. The effect of Concentration Unimodality Constraints. The example uses the “demo_series.mat” file, with the following pre-processing parameters: spectral region cut to 700 – 1630 cm-1; lambda=10,000 and p=0.001; No Normalisation and Savitzky-Golay smoothing with 1st order polynomial and Frame=5. 3 components are selected for MCR-ALS, using automatic initial estimate determination, spectral direction and 10% noise. Convergence limit is set to 1 for speed, and maximum number of iterations is kept at the default 50. Results are shown with no unimodality constraints (a), and concentration unimodality constraints (vertical, b, horizontal, c, and average, d) for all 3 Components. Note that apart from the vertical constraint (b), all results are very similar, but are achieved in much fewer iterations with the constraints.

### Closure

A default spectrum equal length closure constraint is built-in with no user control. It is applied by default, if no normalisation has been performed during pre-processing, and means an equal spectral area normalisation for the MCR-ALS algorithm to avoid possible scale indeterminacies. This is the most common closure constraint for general spectroscopic data. If, however, any kind of normalisation has been applied during pre-processing, the user is assumed to handle scale indeterminacies via that route. In that case NO closure constraint is applied.

Warning/Information: The default spectrum equal length closure constraint is the reason why normalisation is not needed for MCR-ALS. The semi-automatic control (allowing either normalisation during pre-processing or spectral equal length closure but not both) is in place to avoid double normalisations potentially skewing the data.

This kind of semi-automatic closure constraint setting cannot be easily removed or modified in the App without extensive modification of the source code and the MCR-ALS algorithm. Thus, the App is not well suited for datasets that would require no closure or a different kind of closure constraints (such as accounting for mass balance equations or total concentration changes in reaction systems, etc).

## Number of Iterations and Convergence Criterion

The maximum number of iterations and the convergence criterion need to be supplied in the “Iterations” and “Convergence” textboxes of the MCR-ALS Panel of the App. The default values are “50” and “0.1”, respectively. These parameters provide the user with controls to end the MCR-ALS modelling.

The number entered in the “Iterations” box sets the maximum number of iterations that will be performed if convergence or divergence does not happen before (see Perform MCR-ALS).

The convergence criterion should be provided as the percentage of change in the standard deviation of residuals between two consecutive iterations (“0.1 meaning that convergence is achieved when the change in the standard deviation of the residuals is lower or equal to 0.1% between two consecutive iterations). In other words, it sets a threshold for what the user considers as an acceptable fit for the model.

Warning/Information: Low convergence limits are generally harder to meet, especially in only a few iterations. For fine tuning the model, consider low convergence and high iterations. However, it can prove to be unnecessarily time consuming if the model does not change / improve significantly after a certain number of iterations. There is a “Stop MCR-ALS” button to stop the iterations at any moment. For a quick overview, either a high convergence limit or a low number of iterations can be used.

## Perform MCR-ALS

After selecting the number of components (see Number of Components), the direction (Direction), the initial estimates (starting values, see Initial Estimates), optionally setting the constraints (see Constraints) and the end conditions for the modelling (Number of Iterations and Convergence Criterion), the MCR-ALS algorithm can be started by clicking on the “Perform MCR-ALS” button. The button has a red text, to mark that this step provides essential input for other parts of the App, even though they are optional (see Reference Matching and Clustering and Segmentation).

After clicking the button, the algorithm starts and the App updates at each iteration, until the convergence condition is met, the maximum number of iterations is reached (see Number of Iterations and Convergence Criterion), divergence is encountered (see divergence stop Warning/Information below) or the user stops the iterations by pressing the “Stop MCR-ALS” button.

The “Sigma change (%):” text updates showing how the sigma value changes after each iteration (see Number of Iterations and Convergence Criterion for explanation).

The “Lack of fit (PCA %):” text updates, showing the lack of fit (as expressed in percentage of the PCA reproduced matrix) at the current iteration.

The “Variation explained (%):” text updates, showing the percent of variation explained by the model (r2 value of the model) at the current iteration.

Below these texts, a status display continuously updates, informing about the current iteration number and whether the fit is improving or not. When an endpoint is reached (convergence, divergence, maximum number of iterations reached, or the user stopped), the status display will announce the endpoint.

At the bottom part of the MCR-ALS Panel of the App, two plots visualise the results of each iteration, showing the Optimised Spectra and the Optimised Concentrations, respectively. The colouring is kept for each component throughout the MCR-ALS Panel of the App (i.e. if Component 1 was blue in the Initial Estimates plot, it is blue also in the Optimised Spectra and in the Optimised Concentration plots)

Warning/Information: Pay attention to values shown in the text displays as well as in the Optimised Spectra and Optimised Concentration plots to see if the model can / needs to be improved (different constraints and starting conditions have more profound effects, while adjusting the number of iterations and convergence limit can help fine tuning the results or save time).

Warning/Information: Erroneous MCR-ALS models can be easy to discover if MATLAB shows error or warning messages in the Command Window (such as rank deficiencies, NaN values instead of numbers in the results, etc), but they can be hard to spot if the modelling appears to work correctly computationally. Be critical of the results and compare them with known features of the dataset (for instance resolved component maps matching the features in the white light image or not, etc.). The reason for suboptimal models (such as divergence stop) is usually worth investigating, as it can provide valuable information about the dataset.

Warning/Information: Divergence stop. If the fit does not improve in a number of consecutive iterations, it is considered divergence and it will terminate the MCR-ALS process. The number of consecutive iterations that are tolerated without improving the fit before it is considered divergence, i.e. before the MCR-ALS modelling is stopped, is set to 10 in the code. It is an arbitrary number, hard-coded in the App. However, it is easy to modify, simply by changing the value of the “idevmax” variable. It is generally not recommended to have lower numbers, in order to allow the model to escape local minima and find the absolute minimum.

The MATLAB default workspace is updated with 4 variables at the end of the MCR-ALS modelling:

“COpt” contains the optimised concentration profiles, in Number of Spectra * Number of Components format.

“MCRMatrix” contains the input data for the MCR-ALS algorithm (i.e. the dataset as it was passed from the Pre-Processing part of the App, either processed or unprocessed), in Number of Spectra * Number of Spectral Variables (e.g. wavenumbers) format. It does NOT contain the Spectral Variables (Wavenumbers).

“SOpt” contains the optimised spectral profiles, in Number of Components * Number of Spectral Variables (e.g. wavenumbers) format. It does NOT contain the Spectral Variables (Wavenumbers).

“Wavenumbers” contains the Spectral Variables (e.g. wavenumbers, hence the name) in a single row.

Warning/Information: There is no information showing whether the results shown are obtained with the current values or not. For example, if a model was generated using a Vertical mode Unimodality Constraint for the Concentrations for all components, then the user removes this constraint, the results will NOT be deleted until the “Perform MCR-ALS” button is clicked again, which will overwrite the previous results. In short, changing any MCR-ALS parameters does NOT delete the results of an already existing MCR-ALS model, until a new model is created.

Warning/Information: In contrast to changing any MCR-ALS parameters, changing the input data will erase ALL previous MCR-ALS models from the App. This is true both for loading a new dataset (3.1 Loading the Data) and for clicking on either the “Unprocessed for MCR-ALS” or the “Pre-Processed for MCR-ALS” buttons of the Pre-Processing Panel of the App (see 3.12. Passing the Data for MCR-ALS). Variables in the MATLAB Workspace will remain, however, even though they are outdated (i.e. refer to a previous MCR-ALS model, not the currently loaded dataset)

When the MCR-ALS iteration concludes, the Evaluation Panel of the App updates as well, including the folded concentration profiles (concentration maps) of the Components (see Change Plot).

An example is shown in Figure 27.

Figure 27. The updated App after MCR-ALS modelling has finished. The example uses the “demo_image.txt” file, with the pre-processing parameters as in Figure 17, and in the same stage as if the analysis was continued from Figure 20. 3 Components with automatic initial estimate determination, spectral direction and 10% noise. No constraints are applied, the number of iterations is left at the default 50 but the convergence limit is changed to 1 for speed. The purest spectra estimates are marked as well.

## Optimised Spectra

The Optimised Spectra plot continuously updates with each iteration during MCR-ALS modelling. Once the modelling is finished, the plot remains static. The colouring is kept for each component throughout the MCR-ALS Panel of the App (i.e. if Component 1 was blue in the Initial Estimates plot, it is blue in the Optimised Spectra and in the Optimised Concentration plots as well). Hovering over the image will automatically display the context sensitive menu for saving the plot (see Figure 1). It is also possible to zoom in and pan the plot (if spectral features are to be inspected more closely, for example).

Warning/Information: The plot contains all the Component spectra. To save each individual Component spectrum separately, use the “Save Comp Plots” button in the Evaluation panel of the App, or the “Show Comp Plots” button (also in the Evaluation panel of the App) and save the spectra from the resulting pop-up figure manually. The “Show Comp Plots” figure is also the only way to visualise Purest initial estimates in the Concentration direction.

## Optimised Concentrations

The Optimised Concentration plot continuously updates with each iteration during MCR-ALS modelling. Once the modelling is finished, the plot remains static. The colouring is kept for each component throughout the MCR-ALS Panel of the App (i.e. if Component 1 was blue in the Initial Estimates plot, it is blue in the Optimised Spectra and in the Optimised Concentration plots as well). Hovering over the image will automatically display the context sensitive menu for saving the plot (see Figure 1). It is also possible to zoom in and pan the plot (if concentrations in certain spectra are to be inspected more closely, for example).

Warning/Information: The plot contains all the Component concentration profiles. To save each individual Component profile separately, use the “Save Comp Plots” button in the Evaluation panel of the App, or the “Show Comp Plots” button (also in the Evaluation panel of the App) and save the concentration from the resulting pop-up figure manually. Both of these options show (and thus save) ONLY the refolded profiles! Only the Optimised Concentrations plot shows non-folded profiles!

### Show the Component Maps

The MCR-ALS optimised concentration profiles are automatically refolded into a map, in addition to being shown in the Optimised Concentrations Plot. This is most valuable for image datasets, but is not technically limited to those (results from independent spectra or even spectral series datasets can benefit from this kind of visualisation too).

Simply click on the “Show Component Maps” button.

Warning/Information: The “Show Component Maps” button is located in the Evaluation Panel of the App, NOT in the MCR-ALS Panel, as strictly speaking this is NOT a core part of the MCR-ALS algorithm, but is a visualisation option.

The results are shown in a new popup window, that contains two columns and as many rows as the number of Components. The first column shows the refolded Concentration profiles, each with a title “ComponentX Map”, where X stands for the Component number. The second column shows the resolved spectral profiles of the Components, each with a title “ComponentX Spectrum”, where X stands for the Component number.

The refolded concentration profiles are shown in the same colours as the Intensity maps of the Evaluation Panel of the App (see Change Colour), while the Spectral profiles have the same colours as in the MCR-ALS Panel of the App (i.e. if Component 1 was blue in the Initial Estimates plot, it is blue in the Optimised Spectra and Concentration Plots as well as in the Spectral profile of this popup window). If the “Show Purest” checkbox is ticked, the purest spectra are marked in the concentration plots by white diamonds in Spectrum direction, or the purest spectral variable (wavenumber) is shown in the spectral plots with a dashed line in Concentration direction.

To save the plots in the pop-up figure that appears after the “Show Component Maps” button is clicked, hover over the plot and use the appearing context-sensitive icon (see Figure 1).

Warning/Information: Changing the colouring in the Evaluation Panel of the App (see Change Colour), does NOT change the colouring in an already existing pop-up figure. This is an intentional feature, since clicking the “Show Component Maps” multiple times opens up new pop-up windows instead of overwriting the existing one, allowing for testing and comparing different colour schemes for visualisation.

Warning/Information: Refolding (see Change Dimensions (Refold and Transpose)) does NOT change the dimensions of to the Component Maps in an already existing pop-up figure.

Warning/Information: Oversampling (see Oversample) is NOT applied to plots in the pop-up figure.

Figure 28. The popup window showing the MCR-ALS resolved Components, after MCR-ALS modelling finished in Figure 27. Note that the window has its own set of menus and icons (allowing for e.g. saving the plots, zooming in, etc.). The Component spectra colours are matching the colours in the MCR-ALS Panel of the App (i.e. Component 1 is blue, Component 2 is red and Component 3 is yellow, see Figure 27) and the Component maps are using the Colour scheme set by the “Colours” drop-down list in the Evaluation Panel of the App (see Change Colour).

## Save the MCR-ALS Results

There are several ways of saving the MCR-ALS results.

For the generated plots, the following options are available.

- Optimised Concentration and Spectral Plots can be saved by hovering over them and selecting the save option from the appearing context sensitive icons (see Figure 1).

- Individual Concentration Maps can be saved by selecting them from the Visualised drop-down menu and hovering over the Visualised plot for the save icon to appear.

- All Component Maps and their Spectral Profiles can be saved using the menu / icon of the popup window that appears after clicking on the “Show Component Maps” button (see Show the Component Maps).

- The Concentration Map and Spectral plots for each Component can be save using the “Save Comp Plots” button in the Evaluation Panel of the App (Save Comp Plots)

To save the generated data (i.e. the matrices, not the plots), the following options are available:

- Variables in the MATLAB Workspace (see Perform MCR-ALS) can be saved individually by right-clicking on their name in the Workspace pane of the main MATLAB window and selecting “Save As…” in the opening context-sensitive menu.

- Variables in the MATLAB Workspace (see Perform MCR-ALS) can be opened by double-clicking on their name in the Workspace pane of the main MATLAB window, and their content copied and pasted into third party software (such as Microsoft Excel).

However, the above options do NOT save the end conditions of the MCR-ALS modelling.

In order to do that, click on the “Save MCR Results” button. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original dataset was loaded, and suggests a name consisting of the dataset name, appended with the text “_MCRALS_XComp” at the end, where X stands for the number of components the model used (so for a 3 component model, it is MCRResult3).

Saving is possible in .mat and .xlsx formats.

When saving in .mat format, a single file is generated that can be opened/loaded in MATLAB and includes the following 7 variables:

“COpt” contains the optimised concentration profiles, in Number of Spectra * Number of Components format. This is the same as the variable loaded in the MATLAB Workspace at the end of the MCR-ALS modelling (see Perform MCR-ALS).

“Ending” contains a single number referring to how the MCR-ALS optimisation ended. “1” means convergence was achieved, “2” means divergence was encountered, “3” means the maximum number of iterations was reached, and “4” means the user stopped the iteration manually before any endpoint was reached. (To see at which number of iteration the ending happened, see the variable “Iteration”).

“Iteration” contains a single number, referring to the number of iteration when the MCR-ALS stopped (for the reason of stopping, see the variable “Ending”).

“R2Opt” contains a single number, referring to the percent of variation explained, in fraction format (i.e. 0.9975 means 99.75%)

“SDOpt” contains a vector with two values, both describing the lack of fit in relative standard deviation units. The first number is with respect to the PCA reconstructed dataset (i.e. the same number as shown in the App as “Lack of fit (PCA %):”), and the second is with respect to the original data matrix.

“Sigma” contains a single number, describing the last sigma value change during iterations with respect to the experimental data, normalised (It is calculated as sqrt(sum(sum(resn.*resn))/(nrow*ncol)), where “sqrt” and “sum” are the square root and sum functions of MATLAB, “resn” are the normalised residuals of the MCR-ALS optimisation, “nrow” is the number of rows (number of spectra) in the dataset, and “ncol” is the number of columns (number of spectral variables, e.g. wavenumbers) in the dataset).

“SOpt” contains the optimised spectral profiles, in Number of Components * Number of Spectral Variables (e.g. wavenumbers) format. It does NOT contain the Spectral Variables (Wavenumbers). This is the same as the variable loaded in the MATLAB Workspace at the end of the MCR-ALS modelling (see Perform MCR-ALS).

Warning/Information: Importantly, the variables “Wavenumbers” and “Matrix4MCR” are NOT included in the saved .mat file. This is because these variables are not the results of the MCR-ALS modelling, but its input. They can be saved using the “Save Pre-Processed” button of the Pre-Processing Panel of the App (3.11. Saving the Pre-Processed Data), or by saving them directly from the MATLAB Workspace (see options e and f above).

If the results are saved in .xlsx format, a new worksheet with the name “MCRALS_XComp”, where X stands for the number of components. This file contains all the results in an easier to comprehend manner, with titles (such as “MCR-ALS Optimised Spectral Profiles”, etc) included, the data organised, and even spectrum names retained, if there were any at loading.

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file. However, it only overwrites the worksheet within the file that has the exact same name! This allows to e.g. save several different component MCR-ALS models to be saved in the same file, or append the MCR-ALS results to the end of an already existing file, such as the raw input data.

# Visualisation and Classification

The Evaluation Panel of the App provides tools to visualise the results and customise the visualisation. This can be helpful for reporting and / or verifying the results, but it is entirely optional. Some visualisation controls are more suited for images (such as the possibility to load a corresponding White Light Image) and some are more suited for a spectral (e.g. reaction) series or a set of independent spectra (such as the possibility to change the dimensions of the visualisation plots), but most of them are generic (such as changing the colour scheme of the plots, marking spectra for classification, reference matching, etc.)

## White Light Image

A White Light Image is automatically loaded with the dataset if it is located in the same folder as the dataset itself, and has the exact same name as the dataset, with .jpg extension.

When it is not loaded automatically, a White Light Image can be manually loaded by clicking the “Load White Light Image” button in the Evaluation Panel of the App. This opens a standard dialog window, where the file to be loaded can be selected. The default option lists .jpg files, but by choosing the “All Files” option, all image formats supported by MATLAB can be loaded.

## Visualisation Controls

The Evaluation Panel of the App contains at least one concentration map: the original total intensity plot, that displays the original total spectral intensity (without any normalisation) for every spectrum in the dataset. As soon as new concentration maps are generated (see3.7. Integration, or 3.8. Single Point Intensity Evaluation, MCR-ALS resolved concentration profiles of the components, see Perform MCR-ALS and Show the Component Maps, segmentation maps, see k-Means Clustering), they are added to the list of plots, and the latest added / selected is displayed in the App with a title above the plot indicating which one is currently being displayed.

### Change Plot

To change the currently displayed map, use the “Visualised” drop-down list in the Evaluation Panel of the App.

This list always starts with “Orig Tot Int”, which refers to the non-normalised original total intensity plot of the dataset.

The list can contain more items, in the order they were generated, until removed.

The labels “SPInt @XXXXNN”, refer to Single Point Intensity maps, where XXXX is the spectral variable (wavenumber) at which the single point intensity was evaluated and NN is the kind of normalisation performed (see 3.8. Single Point Intensity Evaluation).

The labels “Int @XXXX-YYYYNN”, refer to Integral maps, where XXXX-YYYY denotes the spectral variables (wavenumbers) between which the intensity was evaluated (integral region) and NN is the kind of normalisation performed (see 3.7. Integration).

The labels “CompX” refer to the optimised concentration maps of the MCR-ALS resolved Components, with X denoting the component number. There are as many of these as there were components in the latest MCR-ALS evaluation run.

Labels such as “ClustersX”, where X denotes the number of clusters used for k-means clustering show the result of segmentation.

Warning/Information: The plots and the “Visualised” drop-down list updates as soon as there is new input for it, but NOT otherwise. Old values are kept as long as they are not overwritten / removed. This may result in very obsolete maps! Examples:

Step 1: Single Point Intensity evaluated at 1600 cm-1, with no normalisation, MCR-ALS performed with 3 components, k-means clustering performed with 3 clusters. The “Visualised” drop-down list will contain the following items: “Orig Tot Int”, “SPInt @1600NN”, “Comp1”, “Comp2”, “Comp3”, “Clusters3”.

Step2: Total Area normalisation is selected in the Pre-Processing Panel of the App, and the data is passed on to MCR-ALS, using the “Pre-Processed for MCR-ALS” button. The “Visualised” drop-down list will contain only the following items: “Orig Tot Int” (always present), “SPInt @1600NN” (as the single point intensity has NOT been re-evaluated with the total area normalisation, i.e. the “Evaluate” button in the Single Point Intensity box of the Pre-Processing Panel of the App was not clicked after total area normalisation was selected in the “Norm” drop-down list in the Pre-Processing Panel of the App) and “Clusters3” (no new segmentation was performed). However, all the Component maps are removed!

Step3: A new MCR-ALS model is generated using 4 components. The “Visualised” drop-down list will contain the following items now: “Orig Tot Int” (always present), “SPInt @1600NN” (still no new single point evaluation performed to reflect the total area normalisation), the “Clusters3” segmentation map, and the newly calculated “Comp1”, “Comp2”, “Comp3”, and “Comp4” at the end. Please note that the “Clusters3” map is now very obsolete, as it was built on the results of a 3 component MCR-ALS model which used no normalisation. In other words, it has nothing to do with the current model, but it has not yet been overwritten (i.e. it can still be saved)

Step 4: Integration is performed in the region 1550-1640 cm-1. The “Visualised” drop-down list will now contain the following items: “Orig Tot Int” (always present), “SPInt @1600NN” (still no new single point evaluation performed to reflect the total area normalisation), “Clusters3” (still nothing to overwrite it), “Comp1”, “Comp2”, “Comp3”, “Comp4” and the new “Int @1550-1640TA”.

Warning/Information: If the Purest Markers are shown (the “Show Purest” checkbox is ticked), they are retained when the map is changed in the “Visualised” drop-down list, or when new maps are generated.

Warning/Information: If there are Class Markers shown in the plot, they are also retained when the map is changed in the “Visualised” drop-down list, or when new maps are generated.

Warning/Information: Class markers may be obscured by Purest Markers, as Purest Markers are always on the top layer. However, they can be easily toggled on / off using the checkbox in the MCR-ALS Panel of the App.

### Change Colour

The colour of the visualised plots in the main App window and subsequently opened popup windows (see Show the Component Maps, Change Dimensions (Refold and Transpose) and Oversample) can be changed by the “Colours” drop-down list of the Evaluation Panel of the App.

Warning/Information: Popup windows opened BEFORE the colour change are NOT affected.

Warning/Information: Changing the colour also affects the segmentation map shown in the “Clustering and Segmentation” Panel of the App, since this plot is also part of the visualised plots (can be included the “Visualised” drop-down list)

The default colouring scheme is “Copper” (one of the built-in options of the “colormap” function of MATLAB), as this provides a good range from dark to light and minimum interference with other potential colours in the maps (including Class Markers and Purest Markers) and no confusion with the Component colours (which use the default MATLAB colour sequence, which is currently “Parula”) or centroid profile colours (which are matching the Component colours).

Other colour schemes are all built-in options of the “colormap” function of MATLAB (see MATLAB documentation and Figure 29).

Figure 29. The values of the “colormap” function of MATLAB used by the “Colours” drop-down list. From top to bottom: Parula (default in MATLAB and in spectral plots in the App), Turbo, HSV, Hot, Cool, Spring, Summer, Autumn, Winter, Gray, Bone, Copper (default for Visualisation maps in the App), Pink, Sky (since 2023a) and Jet. See the MATLAB documentation for more details.

### Change Dimensions (Refold and Transpose)

The dimension for the dataset needs to be specified upon loading (see 3.1 Loading the Data). For image data, the X and Y dimensions are set by the image dimensions and should not be changed. Thus, this option is NOT recommended for images.

The default option upon loading (if no other dimensions are specified) is Y=1 and X=total number of spectra, generating a single row in the visualisation plots. This can be suboptimal when a large number of spectra are included in the dataset. Thus, for spectral series and sets of independent spectra, changing the dimensions can help the visualisation and interpretation or tracking / identifying certain samples / spectra. Alternatively, if there is a pattern in the data, this can be retained during visualisation.

As an example, consider a batch experiment, where each batch contains 5 replicates (spectra). Batch 1-2 experiments are performed at pH=3 (one experimental variable), Batches 3-4 are at pH=5, batches 5-6 at pH=7 and batches 7-8 at pH=9. This means 8 batches in total, with 5 spectra in each = 40 spectra. If the data is visualised as 40*1 spectra, it can be hard to see (and mark) individual spectra in the set. However, if the dataset is refolded as 8*5, each batch will have its own row in the visualisation plots. Thus, patters are easier to notice (also beneficial for segmentation) and correlate to features of the dataset. Spectra are also easier to mark. Alternatively, a 10*4 refolding can be helpful as well, in which case every spectra of a certain pH will be shown in one row, with the first half (5 spectra) from the first batch at that pH and the second half (5 spectra) from the second batch at that pH.

The dimension change is entirely optional and the dimensions are left to the user to decide.

Type in the new X and Y dimensions in the respective textboxes in the Evaluation Panel of the App and click the “Refold” button.

To revert to the original dimensions (supplied during loading), click the “Reset” button.

Warning/Information: The new dimensions CANNOT be lower than the total number of spectra (i.e. a dataset with 40 spectra cannot be refolded into a 4*4 dimension, as that only contains 16 elements). In this case, the App will show a warning message but will not change the dimensions.

Warning/Information: The new dimensions are allowed to be higher than the total number of spectra (i.e. a dataset with 40 spectra is allowed to be refolded into a 7*6 dimension, which should contain 42 spectra). In this case, the App will add the extra points at the end of the plot with NaN values. (Using the above example, the plot will contain 6 rows and 7 columns, with the last two points of the last row being blank, and all the rest containing values). This is strictly a display/visualisation tool: NO extra data (spectra) will be added to the dataset (i.e. no new rows in the data matrix).

Warning/Information: Changing the “Visualised” drop-down list does NOT reset the dimension to the original ones as in previous versions of the script.

Warning/Information: Changing the “Colours” drop-down list does NOT reset the dimension, so a new colour scheme can be tested without having to refold again.

Warning/Information: If there are Purest Markers in the plot, their position will be recalculated to match the new dimensions, i.e. they are automatically updated.

Warning/Information: Classes CANNOT be marked in the empty areas of refolded plots (as there is no data there) but can still be selected in the areas that belong to the original dataset.

Warning/Information: The “Show Comp Plots” and “Save Comp Plots” button saves the shows and saves the plots with the current dimensions (i.e. plots with the refolded dimensions can be saved).

Figure 30. Changing the dimensions of the dataset for better visualisation. The example uses the “demo_series.mat” file, and its total intensity plot directly after loading. On top: Accepting the default suggestion of the App upon loading (X=400, Y=1, i.e. all spectra in a single row) creates a plot that is impossible to see. Middle: changing the dimensions to 20 x 20 makes visualisation easier and reveals that the total intensity of the first 17 spectra are considerably lower than the rest.  Bottom: the dimensions can be set to include more data points than the original dataset, but not less. In this example the dimension 20x25 is specified, which equals to 500 data points, although the dataset only contains 400 in total. Thus, the matrix for visualisation is padded with 100 NaNs (not-a-number values) at the end.

### Oversample

The lateral resolution of the generated visualisation plots can be improved virtually, using oversampling. Select the plot for oversampling by the “Visualised” drop-down list (see Change Plot), enter the oversampling factor in the edit field and hit enter. A new pop-up figure appears that contains 2 plots: the original (top) plot as shown in the App, and its oversampled version (bottom). The oversampling factor determines the new number of points (pixels) in the oversampled plot, multiplying BOTH the X and Y points with this number and calculating the values of each point based on the original data points. Example: the demo_image.txt dataset contains a 25*5 image. Using an oversampling factor of 2, the new image will consist of 50*10 points (pixels).

Warning/Information: The oversampling range is limited to 2 – 20.

Warning/Information: The oversampling only visualises the currently active plot. However, new pop-up figures opened by oversampling do NOT overwrite already existing windows, thus several oversampling options can be compared side by side, or several plots can be cycled through and compared / saved.

Warning/Information: Oversampling does NOT change any data matrices, it is only a visualisation tool. Thus, no oversampled data can be saved, only the plots.

Warning/Information: Oversampling can generate artefacts, especially close to the right and bottom edges of the images, and in particular when intensities in the original plot are high in those regions (as there is no data beyond these points to be used for value estimations). Be aware of this limitation of the algorithm.

Warning/Information: Naturally, no Markers (Purest or Class) are included in the oversampling plots, as their position cannot be determined in the virtually enhanced new plots.

Warning/Information: Oversampling refolded data with empty areas will include those empty areas as well.

To perform a new oversampling, click in the oversampling edit field and either change the oversampling value (if a new oversampling factor is to be tested on the same plot), or simply hit “Enter”/”Return” on the keyboard (to keep the same oversampling factor when generating a new plot, after e.g. changing the “Visualised” drop-down list to a new plot, or changing the “Colours” drop-down list to a new colour scheme).

Figure 31. The Oversampling pop-up figure, using the original total intensity plot generated directly after the “demo_image.txt” file is loaded, using an oversampling factor of 10. Top: no oversampling, Bottom: oversampled by a factor of 10. Note the number of pixels in the image (and the resulting smoothness) and the artefacts generated at the bottom and at the right of the oversampled map (drop of intensity). Note also that the window has its own set of menus and icons (allowing for e.g. saving the plots).

### Show Purest

The location of the spectra shown in the “Purest” listbox in the MCR-ALS Panel of the App can be visualised on the plots by ticking the “Show Purest” checkbox. This checkbox is located in the MCR-ALS Panel of the App! White diamond markers with black outlines are added to the plots in the Evaluation Panel of the App (see e.g. Figures 27 and 32).

Warning/Information: Naturally, only those elements of the “Purest” listbox can be shown in the plot that are members of the dataset. Externally supplied Initial Estimates, either manually loaded (see Load Input) ones or those imported from Reference Spectra (see Use Reference Spectra) cannot be visualised.

Warning/Information: The Purest Markers on the visualisation plot are all white (not colour coded) and have no labels to indicate which Component they belong to (see Figure 20 for an example). This information has to be deduced from their position.

Warning/Information: The Markers remain even if the Visualised plot is changed using the “Visualised” drop-down list (see Change Plot) or a different colour coding is selected via the “Colours” drop-down list (see Change Colour), as long as the checkbox is ticked.

Warning/Information: The position of the Markers automatically update when the image is refolded / reset (see Change Dimensions (Refold and Transpose)).

Warning/Information: The Markers automatically update when the number of Components is altered (see Number of Components).

Warning/Information: Markers also appear in the plots of pop-up figures (such as the plots displayed by the “Show Comp Plots” button), except for the Oversampling pop-up figure (see Oversample).

Warning/Information: If the Markers are visible in the Evaluation Panel of the App, they will be visible in the Saved plots as well.

## Show Component Plots

See Show the Component Maps and Change Plot.

## Save Comp Plots

Click on the “Save Comp Plots” button in the Evaluation Panel of the App to save all the component plots individually. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original dataset was loaded, and suggests the same name as the original dataset. The following file formats can be chosen:  .png (24-bit Portable Network Graphics, but no alpha channel (transparency); .jpg (24-bit JPEG); .tif (uncompressed 24-bit TIFF); .eps (Encapsulated PostScript, Level 3 colour); and .pdf (full page Portable Document Format, in colour, including page orientation), .eps and .pdf are vector graphic formats at can be scaled lossless. The others are .img formats with fixed resolutions. EACH PLOT will be saved as an individual file with the selected file format/extension, and the name supplied in the dialog box is used only as a prefix: each file name will be appended by the Component number and the type of plot (Concentration or Spectrum).

Warning/Information: Mac and Windows compatibility and resolution problems of the saved plots may arise. This is especially true for tiff files. Test different formats to find the optimal solution to the current computer architecture.

Warning/Information: Only component maps are saved, Integral, Single Point Intensity, Clusters or the original total intensity plot needs to be saved by selecting them in the Visualised drop-down menu and hovering over their map to bring up the context sensitive icons.

Warning/Information: All files are in the same format. It is not possible to save one Component map as one file type and another as a different one. If that is needed, select the map in the Visualised drop-down menu, hover over the map to bring up the context sensitive icons and save each map in a different format.

Warning/Information: This is the only option to automatically save individual Spectral Profiles.

## Mark Classes

Spectra in the dataset can be assigned to any one of 6 possible classes using radio button selections first: class 1 (black), class 2 (red), class 3 (blue), class 4 (green), class 5 (magenta) and class 6 (cyan). Any particular spectrum can only belong to only one class.

Marked spectra are shown in the Marked Spectra Plot of the Evaluation Panel of the App, using the colour of their classes. The Marked Spectra Plot updates automatically as soon as any Pre-Processing parameter is changed (even if the data is NOT passed to MCR-ALS after changing the pre-processing parameters), or a spectrum is added or removed from the Marked Spectra list.

The positions of the Marked Spectra are also indicated on the Visualised plots, using square markers with their respective class colours and white outlines. These Class Markers remain even if the “Visualised” drop-down menu is changed to show a different map, or the “Colours” drop-down menu is changed to use a different colour scheme for visualisations.

Marked Spectra are also listed in the “Marked” listbox by their spectrum number shown. For MATLAB versions 2023a or newer, the listbox shows the marked pixels in their class colours. For earlier versions of MATLAB, class colours are not retained in the listbox. Spectra are listed in the “Marked” listbox by increasing class numbers first and then increasing spectrum number within each class for MATLAB versions 2023a or newer, and simply in increasing spectrum number for older versions.

Warning/Information: There is no option in the App to mark more than one spectrum at a time.

Marking spectra activates the “Reset All Classes”, “Save Marked Spectra” and “Compile Class Matrix” buttons in the Evaluation Panel of the App. If there are no marked spectra, these buttons are deactivated.

### Manual Marking / Unmarking

To manually mark a spectrum, first select its class by ticking any of the six “Mark Classes” radio buttons, then left-click on the spectrum in the active Visualised Plot to mark it.

Warning/Information: It can be very difficult to accurately mark a spectrum this way if the plot contains many spectra or folded in the wrong dimensions, i.e. pixels are very small. In this case, either use the “Mark from Selected” option, or hover over the Visualised plot and select the Zoom in option (Magnifier glass with a “+” sign in it) and scroll upwards with the mouse until pixels become easily visible / identifiable. Note that clicking anywhere in the plot will mark the pixel, thus panning around is difficult when zoomed in (as it involves click-and-drag).

Warning/Information: If a spectrum is already assigned to a class, it cannot be selected for a new one directly this way (i.e. class membership cannot be overwritten). It first needs to be cleared (unmarked) and then marked to belong to the new class.

To unmark a spectrum of any class, either click on its marker in the Visualised plot, or double click its identifier (the spectrum number shown in the class colour) in the “Marked” listbox (preferred).

Warning/Information: It can be very difficult to unmark a spectrum by clicking on its marker in the Visualised plot, as it requires hitting the invisible boundary around the marker, which is in the bottom right corner of the marked pixel and NOT in its centre. It is considerably easier to use the “Marked” listbox for unmarking.

### Mark from Selected

It can be difficult to manually mark a particular spectrum in the dataset. In this case, the desired spectrum number can be typed in the “Selected Spectrum” edit field in the Pre-Processing Panel of the App. This will show that particular spectrum in the Selected Spectrum Plot (in the Pre-Processing Panel of the App) even if it was not originally included in the “Selected” list (in case the dataset contains more than a 100 spectra, see 3.2 Selecting a spectrum for display). Once the spectrum is shown there, choose the class by ticking any of the “Mark Classes” radio buttons, and click the “Mark from Selected” button in the Evaluation Panel of the App. This will add the selected spectrum to the “Marked Spectra” Plot, its spectrum number will appear in the “Marked” listbox, and its position will be marked in the Visualised Plot with its class colour as well.

Warning/Information: If a spectrum is already assigned to a class, it cannot be selected for a new one directly this way (i.e. class membership cannot be overwritten). It first needs to be cleared (unmarked) and then marked to belong to the new class.

## Reset All Classes

Instead of manually removing all marked spectrum one by one (see Manual Marking / Unmarking), all classifications can be cleared at once. To do this, click on the “Reset All Classes” button in the Evaluation Panel of the App. This will clear the Marked Spectra Plot, remove all Class Markers from the Visualised plots and clear the “Marked” listbox. It will not reset the “Mark Classes” radio buttons, but it will deactivate the “Reset All Classes”, “Save Marked Spectra” and “Compile Class Matrix” buttons in the Evaluation Panel of the App.

Warning/Information: Once deleted, the Markers cannot be recovered and the App gives no warning prior to deleting them!

## Compile Class Matrix

All Marked Spectra can be saved together in a single Microsoft Excel .xlsx file, together with their class information.

Click on the “Compile Class Matrix” button in the Evaluation Panel of the App. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original dataset was loaded. It also suggests a name that contains the original dataset name with “_ClassMatrix” appended to the end. The saved .xlsx file contains the Marked Spectra in the first worksheet: The row contains the spectral variables (wavenumbers), thereafter each row is one spectrum. The first column contains the file names (which include the name of the .xlsx file appended with “SpectrumXX” at the end, where XX stands for the spectrum number), and the second column contains the Class number for each spectrum, thereafter each column contains the intensities at a certain spectral variable (wavenumber).

Warning/Information: The saved spectra cannot be loaded and visualised in App afterwards (note that the button has a black text, not blue) as it contains the class identifiers as well.

Warning/Information: If the Marked Spectra plot is to be saved as well, hovering over the plot and selecting the save option (see Figure 1).

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file when saving the integral results. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Marked Spectra at the end of an already existing file, such as the raw input data.

Warning/Information: In the resulting file, the original spectrum identifiers (spectrum names), if there were any, are NOT retained! Instead the identifiers will consist of the original filename with “SpectrumXX” appended at the end, where XX stands for the pixel (spectrum) number.

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

Warning/Information: The resulting .xlsx file can be directly imported into e.g. Umetrics SIMCA-P. The spectrum names may automatically be marked as OBS IDs in SIMCA-P. Variable IDs, X and Y variables are likely required to be manually specified during importing.

Figure 32. The Evaluation Panel of the App, after spectra are marked for Classes. The example uses the “demo_image.txt” file, after MCR-ALS modelling finished in Figure 27. The “Show Purest” checkbox is ticked so the purest spectra are marked by white diamonds. Classes are marked by black (Class 1), red (Class 2), blue (Class 3) and green (Class 4) boxes with white outlines in the visualised map, and these colours are used in the Marked Spectra plot as well as in the “Marked” listbox, which contains the spectra in class order first, and thereafter spectrum number order (for Matlab2023a and newer). Since there are more spectra marked than space in the listbox, a scroll bar automatically appears on the side of the listbox.

# Reference Matching

The resolved spectral profiles of the Components can be matched to a set of spectra (References) for identification / validation. The reference matching is built solely on simple Euclidean distances (see Match References). The user must always verify the results of the reference matching by visual inspection, the numbers provided in the table are only indicators.

## Load Reference Spectra

Click on the “Load Reference Spectra” button in the Evaluation Panel of the App. A standard dialog window opens to navigate to the folder where the Reference Spectra are located and open the file(s).

There are three ways of providing Reference Spectra input for the App:

- A set of .mat files. Each Reference Spectrum must be in one .mat file, which must contain a single data matrix: the first column is the spectral variables (wavenumbers), the second is the corresponding intensities. The filenames will be used as the names (identifiers) for the reference spectra in the App. Hold “Shift” or “Ctrl” for selecting and loading multiple files simultaneously.

Warning/Information: All files must be in the same folder!

- A set of .txt files. Each Reference Spectrum must be in one .txt file, which must contain two columns (space or tab separated): the first column is the spectral variables (wavenumbers), the second is the corresponding intensities. The filenames will be used as the names (identifiers) for the reference spectra in the App. Hold “Shift” or “Ctrl” for selecting and loading multiple files simultaneously.

Warning/Information: All files must be in the same folder!

- A single Microsoft Excel .xlsx file. All spectra must be in the first worksheet of the file, first column containing the names (identifiers) for the reference spectra (these names will be used in the App) in every even number row, every odd number row containing the spectra variables (wavenumbers) and every even number row containing the corresponding intensities.

Warning/Information: Only one file at a time can be loaded if .xlsx is chosen for input format.

For exact formatting requirements of the Reference Spectra files, see Reference Spectra.

Warning/Information: Only one type of spectra input can be used at a time, i.e. all reference spectra must be either in a set of .mat OR .txt files OR a single .xlsx file, they cannot be combined!

Warning/Information: Reference spectra will be made compatible with the dataset automatically upon loading and will be total area normalised. Compatibility means that reference spectra will be either trimmed to the length of the dataset (i.e. excluding all wavenumbers outside the spectral region of the dataset) or/and expanded to match the length of the dataset (adding zeros, i.e. baseline). In addition, spectral resolution will be adjusted to match that of the dataset, using the built-in “spline” interpolation algorithm of MATLAB.

Warning/Information: Loading any new Reference Spectra will add these to the already existing ones, if there were any. In case of identical names, the reference is considered a duplicate and is NOT added.

Once the Reference Spectra are loaded, the Reference Spectra plot updates to show the loaded spectra, the “Save Normalised Refs” and the “Use Refs as Initials” button become active, as well as the “Match References” button, if MCR-ALS modelling has been completed (i.e. there are resolved spectral profiles to be matched) (Figure 33).

Figure 33. The Evaluation Panel of the App after loading Reference Spectra. The example uses the “demo_image.txt” file, after MCR-ALS modelling finished in Figure 27 and the “demo_references.xlsx” auxiliary data file. Note the conflicting colour scheme of the reference spectra and Components, in e.g. the MCR-ALS Panel of the App (Figure 27).

Warning/Information: Reference Spectra are automatically area normalised upon loading (to match the built-in spectral equal length constraint of the MCR-ALS algorithm (see Closure) in the absence of other normalisation types).

Warning/Information: Reference Spectra are also min-max scaled and offset corrected in the Reference Spectra plot. This scaling is for display purposes only and is NOT used during reference matching.

Warning/Information: All Pre-Processing parameters are ignored by the Reference Spectra, i.e. Reference Spectra are NOT pre-processed! (If they need to be pre-processed by the same parameters as the dataset spectra, save the pre-processing parameters, load the Reference Spectra as if they were a dataset, pre-process them with the saved parameters and save them. This process involves repeated format changes to match different the input requirements of the dataset and the reference spectra, see Input data and Reference Spectra).

Warning/Information: The Reference Spectra plot uses the same colour scheme as the Components in the MCR-ALS Panel of the App. Do not confuse Reference Spectra colours with Component colours!

Warning/Information: The reference spectra are deleted without warning if the spectral region is Cut / Reset, since this renders them incompatible.

## Match References

Click on the “Match References” button to match the MCR-ALS resolved spectral profiles to the loaded Reference Spectra.

Reference matching is based on simple Euclidean distances of two row matrices (the resolved spectral profile of a Component and a Reference Spectrum). The Euclidean distance in mathematics is the straight-line distance between two points in the Euclidean space, and MATLAB defines it as follows:

d = sum((x-y).^2).^0.5, where d is the Euclidean distance, x and y are the two row vectors (the resolved spectral profiles and the reference spectrum, respectively).

The smaller the distance, the closer the points are to each other (better matching).

The direction of the vectors can be determined by the scalar dot product of two real vectors of length n:

u · v=u1v1+u2v2+...+unvn .

If the dot product is equal to zero, then u and v are perpendicular. This relation is commutative for real vectors, such that dot(u,v) equals dot(v,u).

Once the Reference Match is performed, the results are shown in two pop-up windows; one contains a Table with the numerical results (Figure 34), the other contains as many plots as there are Components in the MCR-ALS model: each plot showing one resolved Component (in thick black lines) and all Reference Spectra in the same colours as in the Reference Spectra plot (Figure 35). The Table will mark matches over 90 percent with a green background. Matches over 98 % will also have a success checkmark icon appended. However, the match percentages shown in the Table of the popup window should ONLY be used as indicators and visual inspection of the spectral profiles is ALWAYS necessary and should be trusted over the numeric results (Figures 34 and 35). This is especially true for complex spectra with diagnostic bands. Consider the following two hypothetical scenarios:

- A resolved Component contains a single band between 515-525 cm-1 and the Reference Spectrum a single band between 1550-1560 cm-1. If the spectral range is from 400-2000 cm-1 and the spectral resolution is 1 cm-1, there are 1600 cm-1 points in each spectrum. Of these, only 20 points are not agreeing (10 in the 515-525 cm-1 region, and 10 in the 1550-1560 cm-1 region), and 1580 points are matching completely. Thus the Euclidean match will be excellent, whereas spectroscopically it is clear that the match should be 0%, since both spectra feature only one band (diagnostic), and they do not match. This false positive match is provided by matching the absence of bands (baseline), which is indeed a valuable information, but in this case numerically outweighs the information of the diagnostic bands.

- The resolved Component and the Reference Spectra contain the EXACT same spectra, but due to experimental errors (such as improper/lacking calibration), one spectrum is shifted by 10 cm-1. Since all bands are shifted, this can result in a relatively poor Euclidean match (not even the baselines are matching), whereas a visual inspection of the spectral profiles reveals the presence of all diagnostic bands in both spectra, the only difference being the shifted position, indicating a potential calibration error.

In real life, these extremes do not occur, but the presence/absence of diagnostic bands is hard to quantify numerically (case 1), and the positions of diagnostic bands can easily shift (e.g. due to different chemical environments, pH, etc) even though the same compounds are recorded (case 2).

Warning/Information: The Reference Spectra plot uses the same colour scheme as the Components in the MCR-ALS Panel of the App. Do not confuse Reference Spectra colours with Component colours (see Figures 27, 33 and 35)!

Warning/Information: The matches (Euclidean distances) will be converted automatically to percentages for easier interpretation. Values below 0.01 (less than 1% match) will be replaced by 0. Otherwise values will be rounded to whole percentages.

Warning/Information: Keep in mid that Reference Spectra are automatically area normalised upon loading (Load Reference Spectra).

Warning/Information: All Pre-Processing parameters are ignored by the Reference Spectra, i.e. Reference Spectra are NOT pre-processed! (If they need to be pre-processed by the same parameters as the dataset spectra, save the pre-processing parameters, load the Reference Spectra as if they were a dataset, pre-process them with the saved parameters and save them. This process involves repeated format changes to match different the input requirements of the dataset and the reference spectra, see Input data and Reference Spectra). Consider this carefully if any kind of normalisation has been performed to the dataset, as this can skew the match results. Due to the built-in spectral equal length constraint (see Closure), it is recommended NOT to use any normalisation for the dataset, and this is especially important if Reference Matching is to be performed.

To save the Reference Matching plots in the pop-up figure, use the menu or the icons of the pop-up figure itself (see Plots).

To save the Table containing the numerical results of the Reference Matching, use the “Save Match Results” button (see Save Match Results). This will only save the alphanumeric data (i.e. names and numbers), not the styling (background colour, success checkmark icon).

Warning/Information: The Table showing the numerical results of the Reference Matching may not be fully visible in the popup window, depending on the number of Components and Reference Spectra and their names. However, slider / scroll bars should be visible to allow for scrolling to different parts of the Table. In addition, MATLAB allows for resizing the pop-up window, which should resize the Table as well.

Figure 34. The numerical results of Reference Matching, as shown in the pop-up window. The example uses the “demo_image.txt” file, after MCR-ALS modelling finished in Figure 27 and the “demo_references.xlsx” auxiliary data file. Note that the window has its own set of menus and icons. Resizing can change the window, but changing Table dimensions can be difficult. A scroll bar appears if the table does not fit in the frame. Note that Component 1 matches the “lignin” reference spectrum well, but Component 3 is almost equally matching the “cellulose” and “lignin” reference spectra. See also Figure 35.

Figure 35. The graphical results of Reference Matching, as shown in the pop-up figure. The example uses the “demo_image.txt” file, after MCR-ALS modelling finished in Figure 27 and the “demo_references.xlsx” auxiliary data file. Note that the window has its own set of menus and icons, allowing for e.g. saving the plots. Note that Component 1 matches the “lignin” reference spectrum well, as noted from the tabular results as well (see Figure 34) but it is not entirely “clean” (bands between 1050 – 1150 cm-1 are more similar to cellulose, for instance). Component 3 has significant contributions from both “cellulose” and “lignin”. This indicates that the current MCR-ALS model was unable to resolve pure lignin and cellulose spectral profiles in this dataset with these processing parameters.

## Save Match Results

The numerical results of the Reference Matching can be saved by clicking on the “Save Match Results” button in the Evaluation Panel of the App. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original dataset was loaded. It also suggests a name that contains the original dataset name with the number of components used in the MCRALS model and the text “_RefMatching” both appended. The results will be saved in “ReferenceMatches” worksheet of the file.

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file when saving the integral results. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Reference Match results at the end of an already existing file, such as the raw input data.

Warning/Information: The spectra used for Reference matching are NOT included when the results are saved. These have to be saved / added separately (see Save Normalised References)

Warning/Information: .xlsx files are much slower to read and write than .mat files. Please wait until the save process finishes.

Warning/Information: The matches are shown in whole percentages (rounded). Values below 0.01 (less than 1% match) will be replaced by 0.

Warning/Information: Only the Table contents are saved, the formatting / styling settings are not (no green background or success checkmarks).

## Save Normalised References

Since the Reference Spectra are automatically made compatible and are area normalised upon loading, they are not identical to the original spectra that were loaded. Thus, they may need to be saved along with the resolved spectral profiles (Save the MCR-ALS Results) and the results of the Reference Matching (see Match References and Save Match Results), for complete documentation or use by third party software.

Click on the “Save Normalised References” button in the Evaluation Panel of the App. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original Reference files were loaded. Irrespective of the input format (see Reference Spectra and Load Reference Spectra), the normalised Reference Spectra will be saved as a single Microsoft Excel .xlsx file. The App suggests a name that contains the original dataset name appended by the text “_NormRefs” The normalised reference spectra will be saved in the “NormReferences” worksheet of the file.

Warning/Information: If an Excel file with the same name already exists in the same folder, a warning message appears asking whether or not to overwrite the file when saving the integral results. However, it only overwrites the worksheet within the file that has the exact same name! This allows to append the Normalised Reference Spectra at the end of an already existing file.

The saved .xlsx file contains all normalised Reference in the same format as used for Excel file input (), hence the blue text of the button. However, Excel inputs ONLY read the first worksheet of a file, and thus if Reference Spectra are to be loaded this way by the App, the “NormReferences” worksheet needs to made the first in the file.

## Use References as Initials

See Use Reference Spectra.

# Clustering and Segmentation

The maps generated by the intensity evaluations (see 3.7. Integration and 3.8. Single Point Intensity Evaluation) or by mapping the concentration profiles of MCR-ALS resolved Components (see MCR-ALS, Visualisation and Classification and Show the Component Maps) often show gradients within the sample, without clear / sharp boundaries. Different segmentation methods are available to generate maps with sharp boundaries and the App uses k-means clustering (see k-Means Clustering). The clustering is based on the concentration profiles of the MCR-ALS resolved Components only (no clustering can be done in the App using integrals or single point intensities) and it is generally useful for image datasets, i.e. identify chemically distinct zones in the image. However, clustering can be useful for spectral series or even for a set of independent spectra, especially when they are ordered in chemically / experimentally meaningful sets instead of randomised sequences.

## Number of Clusters

The number of clusters must be supplied in the “# Clusters” edit field in the Evaluation Panel of the App before k-means clustering can take place. There are two ways of adding the number of clusters:

### Manual Input

Simply type in the desired number in the “# Clusters” edit field in the Evaluation Panel of the and hit “Enter/Return”.

Warning/Information: The minimum number of clusters is 2 (default in the App upon Loading) and the maximum is the number of total spectra in the dataset – 1. However, it is not meaningful to use a number of clusters so close to the total number of spectra (as essentially every spectrum would become its own cluster).

Warning/Information: Manual input is ideal for experimenting with different numbers of clusters.

### Silhouette Clusters

If there is no a priori information available regarding the possible number of clusters, silhouette clustering can be used to explore the dataset. In MATLAB, the silhouette value for each point is defined as a measure of how similar that point is to points in its own cluster, when compared to points in other clusters. The silhouette value for the ith point, Si, is thus calculated as

Si = (bi-ai)/max(ai,bi), where ai is the average distance from the ith point to the other points in the same cluster as i, and bi is the minimum average distance from the ith point to points in a different cluster, minimized over clusters. This distance in the App is based on Euclidean distances (for a definition, see Match References), which is the default setting of MATLAB (although MATLAB allows for any supported distance metric to be used for the silhouette evaluation).

The resulting silhouette values range from -1 to +1. A high silhouette value (i.e. close to +1) indicates that the point (spectrum in the dataset) matches to other points (spectra) in its own cluster very well, and is rather dissimilar (matches poorly) to points (spectra) in neighbouring clusters. If most points have a high silhouette value, then the clustering solution is considered appropriate, otherwise the clustering solution may have either too many or too few clusters.

When the user clicks on the “Silhouette Clusters” button in the Evaluation Panel of the App, the script starts k-means clustering with 2 clusters, and increases the number of clusters by 1 in each loop until the silhouette value improves. During this time, a dialog pop-up appears, telling the user to wait until the process finishes. The number of clusters that gives the best silhouette value is considered the appropriate clustering solution, and this number is shown in the “# Clusters” edit field when the silhouette clustering process is finished (and the wait dialog automatically closes).

Warning/Information: Silhouette clustering is not fail-proof and is not an iterative optimisation procedure, but a simple loop. Feel free to override the silhouette cluster number manually, if needed (a priori knowledge of the dataset) or for exploratory purposes (testing different cluster numbers)

Warning/Information: Silhouette clustering is normally fast, but in case of large datasets (many Components and many spectra) on slow computers it may take several seconds. Please wait until the wait dialog pop-up disappears, otherwise plots in the App may render incompletely or not at all.

## k-Means Clustering

Click on the “k-Mean Clustering” button in the in the Evaluation Panel of the App to perform segmentation by k-means clustering, based on squared Euclidean distances and MATLAB’s built-in k-means++ algorithm for cluster centre initialisation. The number of times to repeat the clustering using new initial values is determined by the ‘Replicates’ options in the source code of the App. It is set to 50 by default, as it is relatively fast yet allows for clusters to be stable (i.e. making sure the initial values do not influence the end results. If the Replicates value is low, cluster numbering could jump, i.e. one spectrum could belong to cluster 1 during one clustering, and to cluster 2 in the next. This does not mean that the segmentation boundaries would change, i.e. the same spectrum would always belong to the same cluster, only the number of that cluster would change. In practical terms, this would mean that the colouring of the segmentation map would change every time clustering is done, although the pattern would remain constant. This can be confusing and irritating).

Warning/Information: Although the number of Replicates is hard-coded in the App, it is easy to change in the source code. Look for the line “[app.IDX, app.Centr] = kmeans(DataIn, app.ClustersEditField.Value, 'Replicates', 50);” and change to the desired number.

When clustering is finished, the Segmentation Map and the corresponding Centroid Profiles are automatically updated in the App (see Figure 36). The Segmentation Map shows the cluster distributions in the dataset, folded to the same dimensions as all other Visualised plots and in the same colour scheme as set by the “Colours” drop-down list in the Evaluation Panel of the App (see Change Colour). The Segmentation Map is also added to the end of the list of plots in the “Visualised” drop-down list of the App (see Change Plot). The Centroid Profiles display a stacked bar plot, showing the contribution of each Component to each cluster, in percentage. The colouring in the Centroid Profiles matches the colour coding of the Components throughout the App (see the Legend of the Initial Estimates Plot in the MCR-ALS Panel of the App, Initial Estimates).

Figure 36. The Evaluation Panel of the App. The example uses the “demo_image.txt” file, after MCR-ALS modelling finished in Figure 27 and Silhouette Clusters, resulting in 3 clusters. The Segmentation Map uses the default “Copper” colour scheme and the Centroid Profiles use the Component colours matching the MCR-ALS Panel of the App (i.e. Component 1 is blue, Component 2 is red, Component 3 is yellow, see e.g. Figures 20 and 27). The Segmentation map identifies the lumen areas of the image as cluster 3 (beige colour), and the Centroid profiles indicate that spectra in these regions are low in intensity and dominated mostly by Component 3. Cluster 2 (brown) matches the location of the middle lamella, and accordingly the centroid profiles show a high contribution of Component 1 to this cluster. Component 1 is mostly lignin, as indicated by Reference Matching (see Figures 34 and 35). Finally, Cluster 1 (black) is the cell wall, with lignin and cellulose as main constituents (Components 1 and 3, blue and yellow, see Centroid Profiles and Figures 34 and 35). Note that while the resolution provided by this MCR-ALS model is suboptimal (see Figure 35 caption), the segmentation map is still valuable for identifying chemically different regions of the image.

Warning/Information: There can only be one plot with the same name at any given time in the “Visualisation” list, which means that if a new Segmentation Map is generated with the same number of clusters, the old one gets overwritten. However, the Segmentation Map and the corresponding Centroid Profiles are ALWAYS updated whenever a new one is generated. In other words, while different segmentation maps can be retained in the Visualised plots, their corresponding centroid profiles are lost.

Warning/Information: If a Segmentation Map / Centroid Profile plot needs to be saved, hover over the plot and select the appropriate save option from the context sensitive icon set that appears. Alternatively, use the “Save Segmentation Results” button to save the data (see Save Segmentation Results) from which the plots can be re-generated manually. (Note that this button has a black text, meaning that its saved data cannot be directly read by the App).

Warning/Information: The Segmentation Map is also added to the list of “Visualisation” plots. Thus, it can also be saved via that route too. Use that option if the Pure Markers (see Error! Reference source not found.) or Class Markers (see Mark Classes) need to be visible on the saved Segmentation Map, as those are not shown in the Segmentation Map plot.

Warning/Information: Since the Segmentation Map is also added to the list of “Visualisation” plots, it can be refolded (see Change Dimensions (Refold and Transpose)) and saved with the new dimensions via that route.

## Save Segmentation Results

Click on the “Save Segment Results” button in the Evaluation Panel of the App to save the results of the segmentation in a single .mat or .xlsx file. A standard save dialog window appears where the folder and filename can be provided.

By default, the App suggests the same folder from which the original dataset was loaded. The App will also suggest a name that contains the original dataset name, appended by the text “_ClusteringResultsXComp_YClusters”, where X is the number of Components the MCR-ALS model had and Y is the number of Clusters used for k-means clustering.

Warning/Information: If a file with the same name already exist in the target location (folder), a warning message is shown. In case of .mat files, they will be overwritten. In case of .xlsx file, however, only the worksheet with the exact same name (“ClusterinResults”) will be overwritten! This allows to append the Clustering results at the end of an already existing file.

The saved single .mat file will contain two MATLAB variables:

“Centr” and “IDX”. “Centr” stands for the centroid profiles, i.e. the contribution of each resolved component to a certain cluster, and thus consisting of as many rows as there are clusters and as many columns as the number of MCR-ALS resolved components. “IDX”, contains the cluster identifiers, i.e. a single number to denote which cluster each spectrum in the dataset belongs to. Thus, it consists of a single column with as many rows as there are spectra in the dataset.

Warning/Information: The .mat file does NOT contain XY dimension information (for image data sets) or filenames (for spectra series). The XY dimension can be added to the filename upon saving (default suggestion by the App) but spectrum names must be saved separately for spectra series.

Warning/Information: Only the data matrices will be saved, the plots will not be included!

In case of .xlsx files, the saved results will have descriptive headings, and the initial spectrum identifiers (names), if there were any, are also retained for the IDX list (i.e. showing which cluster each spectrum belongs to).

# Closing the App

The App can be closed by simply closing the main App window or by exiting MATLAB. In both cases, a warning dialog appears, informing the potential loss of all unsaved data. Proceed accordingly.

Warning/Information: If only the main App window is closed but MATLAB is not, some data might still be recovered. In particular, the variables that are placed in the MATLAB Workspace after an MCR-ALS modelling has been performed will remain there after the main App window is closed (see Perform MCR-ALS). These variables are easily accessible. Other variables may be possible to salvage, but they would require expert computer skills.

Warning/Information: Closing the main App window does NOT close any potential pop-up window opened by the App (such as Component Maps or Reference Matching results, etc.). Thus, the contents of these windows are possible to save even after the main App window is closed, until the user manually closes them, or exits MATLAB.

Warning/Information: Closing the main App window does NOT clear the variables in the memory. Type “clear all” in the MATLAB Command Window to clear all variables from the memory. There are no warnings, but this will clear all variables permanently, with no possible recovery options.

Figure 37. The fully populated App. The example uses the “demo_image.txt” file after MCR-ALS imaging (as described in Figures 17 and 27), with the “demo_references.xlsx” auxiliary dataset loaded. Classes are marked as in Figure 32, and segmentation performed after silhouette clustering with 3 clusters (Figure 36).

# Acknowledgements

The Vibrational Spectroscopy Core Facility is supported by the Chemical Biological Centre and the Department of Chemistry at Umeå University.

The contributions of A. de Juan, J. Felten, T. Gillgren, K. Hagvall, H. Hall, J. Kenney, J. Jaumot, J. Loring and P. Persson to writing, designing and/or testing parts of this App are greatly acknowledged. Discussions with A. de Juan and her comments are highly appreciated for the MCR-ALS part of the App. Discussions with J. Kenney and J. Loring have been especially valuable for designing and implementing certain functionalities of the App.